Needle Detector Calibration Log

Factory Name: ……………………………………….

Remaining needle detector must verify the calibration and accuracy once every 2 hours in a working day by using the 0.8 to1.2 mm sphere ferrous card to check machine sensitivity.

CROSS SECTION NEEDLE DETECTOR TUNNEL

TOP			A7		A8		A9

CENTRE		A4		A5		A6

BOTTOM		A1		A2		A3

- Switch on the machine and adjust sensitivity level to less than 0.8/1.00/1.2 mm sphere as per buyer requirements.

- Position the 0.8/1.00/1.2 mm sphere ferrous check card onto the conveyor belt at position A1 let the tats cards passes through the search head.

- The needle detector should be activated (alarming).

- Report the procedure 1-3 with position the 0.8/1.00/1.2 mm sphere ferrous card onto the belt at position A2 & A3.

- The needle detector should be activated (alarming) also.

- Test A1, A2 &A3 with 0.8/1.00/1.2 mm ferrous check card only. No garment or paper is allowed to put together during testing.

- Report the procedure 1-3 with position the 0.8/1.00/1.2 mm sphere ferrous check card on the top of he stand, and placed the stand onto the conveyor belt at position A4, A5, A6, A7, A8 & A9 and record.

|  |  |  | POSITION |  |  |  |  |  |  |  |  | Operator’s Signature |  | FQA/FSSS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Time | Buyer | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 |  |  | Date | Signature |

“Y” The needle detection activate (alarming) when 0.8/1.00/1.2 mm ferrous check card pass through the search head “N” The needle detector reaction (not alarming) when 0.8/1.00/1.2 mm ferrous check card pass through the search head.

Production Safety Hand Book

QC		FINI-M		Q.M		SR.F.M	P.M		Q.A.M		F.M

Quality Department