Prepared & Revised by:-

Version Number   :

Revision Number  :

Rivision Date          :

Issue Daate             :

Approved By

Executive Director

QUALITY POLICY

………………. Limited is committed to achieve optimum customer satisfaction with best quality readymade garments with safe and legal requirements through specified & effective Quality Management System, whilst ensuring that all customer obligations and requirements are met.

----------------- wjwg‡UW wba©vwiZ I Kvh©Ki gvb e¨e¯’vcbv c×wZi gva¨‡g wbivc` I AvBb m¤§Z Dcv‡q m‡ev©Ëg gvbm¤úbœ ˆZix †cvkvK mieivnKi‡Y †µZvi m‡e©v”P mš‘wó AR©‡b e× cwiKi hvnv‡Z mKj †µZvi Pvwn`v I eva¨evaKZv c~iY nq|

wbev©nx cwiPvjK

Purpose:

This Quality Assurance Manual is prepared to ensure proper communication, implementation and maintain unique Quality System at various / different operations for manufacturing Ready-Made Garments in order to achieve desired level of Quality of the product of our valued customers and finally to obtain customer’s satisfaction.

Scope:

This Quality Assurance Manual applies to all manufacturing processes, area & section of ……………………….. Limited, South Shyampur, Hemayetpur, Savar, Dhaka.

Distribution of Quality Manual:

- Chairman –

- Managing Director –

- Executive Director –

- Head of Quality -

- Sr. Manger Compliance -

Availability of Q A Manual:

The original copy of the Quality Manual will be under custody of Head of Quality or Sr. Manager (Compliance). For changing full or any part of the manual to meet the customer / legal requirement, any suggestion, recommendation with regard to modify & change any part of the manual should be referred to Head of Quality and Senior Manager Compliance of

DIRD Group – Savar Zone. The revision, include / exclude any part of the manual should have to develop through QMS Meeting.

Table of Contents

| SL | Content | Page |
| --- | --- | --- |
| 00 | QA MANUAL (Front Page) | 01 |
| 00 | QUALITY POLICY | 02 |
| 00 | PURPOSE, SCOPE & Distribution of Quality Manual | 03 |
| 00 | QUALITY MANUAL AVAILABILITY | 03 |
| 00 | TABLE OF CONTENT | 04-05 |
| 00 | Quality Assurance ORGANOGRAM | 06 |
| 01 | Responsibilities and Work Devision (Head of Quality) | 07 |
| 02 | R & D / IE System & Procedure | 08 |
| 03 | Quality Control System & Procedures | 08 |
| 04 | Fabric Inspection system | 10 |
| 05 | Pre Production (PP) Meeting | 12 |
| 06 | Participation of PP Meeting | 13 |
| 07 | Pilot Run (PPR) | 13 |
| 08 | Cutting Quality Management System | 13 |
| 09 | Sewing Quality Management System | 14 |
| 10 | Quality Control of Subcontractors / Suppliers (Printing, Embroidery) | 15 |
| 11 | Quality Assurance (Finishing section ) | 16 |
| 12 | Defect Codes Classification | 19 |
| 13 | Standard Operating Procedure (SOP) of soiling control | 19 |
| 14 | Section / Process wise Quality Policy & Procedure | 19 |
| 15 | Procedure for children’s Products | 20 |
| 16 | Fabric Relaxation policy | 20 |
| 17 | Policy for Popper, Rivet, Press Stud & Button attachment | 21 |
| 18 | Operating procedure of Fusing Machine | 21 |
| 19 | Complaint / Feedback Handling Procedure | 22 |
| 20 | Product Recall / Withdrawal Procedure | 24 |
| 21 | PROCEDURE FOR PRODUCT CLAIMS | 24 |
| 22 | PROCEDURE FOR LABEL MANAGEMENT ( SEWING ) | 25 |
| 23 | PROCEDURE FOR TRIMS /ACCESSORY INSPECTION,HOUSE KEEPING & SYSTEM | 25 |
| 24 | Procedure for Children’s Products | 25 |
| 25 | PROCEDURE FOR HAND SEWING NEEDLE & TOOLS CONTROLS | 26 |
| 26 | PROCEDURE FOR KNIFE BLADE CONTROLS | 26 |
| 27 | PROCEDURE FOR LABEL MANAGEMENT IN WAREHOUSE | 26 |
| 28 | PROCEDURE FOR POPPER, RIVET, PRESS STUD & BUTTON ATTACHMENT | 27 |
| 29 | Procedure for Measurment Audit | 27 |
| 30 | PROCEDURE FOR PRE-SHIPMENT AQL | 27 |
| 31 | PROCEDURE FOR RAW MATERIAL APPROVAL | 28 |
| 32 | PROCEDURE FOR RAW MATERIAL INSPECTION & TESTING | 28 |
| 33 | PROCEDURE FOR RAW MATERIAL CONTROL | 28 |
| SL | Content | Page |
| 34 | PROCEDURE FOR USED NEEDLE CONTROL | 29 |
| 35 | Guideline of Mold Issue | 29 |
| 36 | Product Withdrawal & Recall Procedure | 30 |
| 37 | Raw Material Supplier Approval/Selection Procedure | 34 |
| 38 | Raw Material Supplier Performance Monitoring Procedure | 34 |
| 39 | Risk Assessment for Children Product | 35 |
| 40 | Warehouse Cleaning Procedures | 39 |
| 41 | Reject Disposal Policy & Procedure | 39 |
| 42 | Needle & Sharp Tools Control Policy | 40 |
| 43 | Pest Control Policy | 42 |
| 44 | Emergency Accident & Incedent Management | 44 |
| 45 | Product Safety Legality & Quality Issues | 45 |
| 46 | Schedule Internal Audit | 45 |
| 47 | Management Review Meeting on QMS | 45 |
| 48 | Risk Assessment Review Meeting | 46 |
| 49 | Standard Operation Procedures | 46 |
| 50 | PRODUCTION PROCESS FLOW DIAGRAM | 54 |
| 51 | Forms & checklist | 57- |

Chairman

Head of Quality

Executive Director

Quality Manager

QCI Sewing

QCI Finishing

QCI Cutting

QC Cutting

QI Cutting

QC Sewing

QI Sewing

QC Finishing

QI Finishing

QCI Laundry

QC Laundry

QI Laundry

QA

Auditor

Finishing

QA

Auditor

Sewing

Approved By

Col. K A B Mainuddin (Retd.)

Executive Director

DIRD Group – Savar Zone

1. Responsibilities and Work Division (Head of Quality)

- Develops quality standard aligned to buyer specifications and defines tolerance limits and QC routines for ensuring 100% in process quality.

- Assumes overall responsibility for quality assurance with safety & legality of raw materials and finished goods in all the stage of manufacturing process (garment, print, embroidery, wash etc) and reporting and analysis of deviation against pre-set specification.

- Coordinate with marketing to obtain customer feedback on quality and initiates steps to incorporate it into the quality assurance process.

- Develops and implements the quality assurance strategy of Dird Group to cater to buyer defined quality norms.

- Identifies opportunities across the supply chain and drive process improvement through quality control parameters to enhance in process to quality standards.

- Design and organizes company –width quality awareness initiatives.

- Coordinate with marketing department/merchandising executive for buyer specification and quality guidelines.

- Develop internal quality assurance guideline and process standards based on buyer specification and communicate to production, P&D, planning, stores and marketing department.

- Ensure 100% quality assessment through appropriate statistical sampling of raw materials and finished products leading to minimum rejection.

- Ensure documentation of quality parameters and variations from norms for all processes and communicates the same to the concerned departments.

- Follow up up with unit QA Manager and all the employees under his quality supervisor for reporting and documentation.

- Ensure revision and modification of quality norms based on ck=hanging best practices, industry trends and buyer requirements.

- Analyses data from production units to explore reason for rejection rate: reworks; low productivity. and use the same to provide guideline and suggestion to production  units on appropriate measures  and standardized manufacturing process for ensuring 100% quality.

- Conduct periodic production floor QC audit to ensure adherence to technical specification and norms provide to production units.

- Coordinate with production for qc during size set sample development pre production sample and before bulk cut for necessary approval.

- Coordinates with merchandizing and production personnel during buyer QA and final inspection to document feedback on quality standards.

- Plan s QC manpower development for all manufacturing stage in consultation with the concerned department head, plan head, and IE head.

- Coordinate with key people internally to ensure strict adherence to quality assurance schedules and reporting.

- Coordinate with manager planning and production head to re-allocate resources in case of change production schedule of work allocation.

2. R & D / IE System & Procedure:

Before starting bulk production, a few pieces of garments are made in actual production conditions and system using actual fabrics. Investigations are made during making of the garment by the Q.A. in assistance with the R&D/IE team. The various aspects to be looked upon during trials are as follows:

- Styling, fabric quality, like grain line, slit line, GSM, elasticity, whiteness, shrinkage, etc.

- Recommended processing/finishing of fabric.

- Machines recommended for production of the garments.

- Attachments, guides to be used in sewing machines.

- Construction problems and preventive action to be taken.

- Embroidery recommendations and design problems.

- Recommend garments' finishing system.

3. Quality Control System & Procedures:

## Raw materials inspection and testing policy

## Minimum 10% fabric rolls to be picked from lot for audit.

- Fabric inspection should be carried out on AQL 4 Point system.

- All rolls shade continuity should be maintained.

- Fabric inspection machine should be placed in a suitable area and there lighting should be 750-1000 LUX ( or as per buyer specified standard)

- Every lot should be check for shrinkage, GSM, Spirality.

- Make mock up to check trims shade & fabric temrapture test.

- All record should be maintained.

## Construction of Woven Fabric:

- Ends/Inch

- Shots/Inch

- Oz/ Sq.yd. (Weight of fabric)

There are two methods of defining fabric construction

- British system

- Metric system

British system: In this system both ends and fill, warp and weft count are defined in British system.

Example: Mono color plain fabric.

32” (warp count) X 40” (weft count)

72(ends/inch) x 64 (fill /inch)

Metric system:-In this system ends and fill, warp and weft are expressed as Tex and C.M

Example: - Fine cotton gabardine

15/2 (warp Tex) x 15/2 (weft Tex)

43 (ends/C.M) x (30 fill/cm)                     Here 2 indicates 2ply.

Conversion

Ends/Inch= Picks per C.M x 2.54, Fill/Inch=Fill per C.M x 2.54

In British system weight of fabric is expressed as oz per square yds in short oz/sq.yd

In Metric system weight of fabric is expressed as gms/sq meter or simply g.s.m.

Types of wave

- Plain weave

- Twill weave

- Sateen weave

- Stain weave

Divisions of Fabric

a) Plain Weave                         b) Twill Weave

- Oxford	                                       i)  Denim

- Semi Oxford                                    ii)  Jean

- Coated                                             iii) Gabardine

Some Terms Related to Nylon Fabric, e.g. 210T 70/34 SD

Yarn

Ends= 100

Picks=110

Denier=70

Filament, filament count=34

Denier per per filament70/34=dpf

Luster= British; SD semi-dull; full dull

Polymer = Nylon6, 6.6.

Defect of Fabric:

- Yarn Fault

- Slub

- Thick and thin

- Yarn contamination

- Hairiness

- Construction Fault

- Broken ends

- Mis pick

- Holes

- Reed mark

- Dent mark

- Loom bar

- Knots

- Selvedge defects

- Snarl/Turn back

- Float

- Dyeing Fault

- Dye/coating spot

- Color shading

- Crease

- Streak

D) Finishing

- Bias and Boa

- Fold mark

- Narrow for defective selvedge

E) Cleanness

- Stain

- Oil/Crease mark

- Dirt/soil mark

- Water mark

- Abrasion mark

Fabric Inspection:

- 4 PTS FABRIC INSPECTIONS

- BLANKET & SHRINKAGE CHECK

- SHADE SEGREGATION

- TWISTING CHECK

- SHADE CONTINUITY CARDS

- RUNNING SHADE CHECK

- 100% FABRIC RELAXATION-24 HRS

- SHRINKAGE & SHADE WISE CUTTING

Acceptance Criteria: Points / 100 Sq. Yards:

Grade  ‘A’ 		Less than 20 penalty 		points- Good Quality

Grade  ‘B’  		20 to 30 penalty 		points- Acceptable Quality

Grade ‘C’ 		above 30 penalty 		points-Reject or approved on deviation as per NC procedure. (Before cutting prior approval need from Authorized Person/position).

## 4. Fabric Inspection system:

Objective:  The aim of fabric inspection is to check quality, quantity and packing by visual, measurement and color check conforming to specification and general appearance of the approval sample given by buyer.

Inspection procedure:

- Receive inspection advice

- Receive approval color and quality swatches

- Check lot size and label against the submitted packing list and inspection advice.

- Randomly draw 10% of the lot or sample size as per US-MIL-STD-105E.

    - Shipment 	 less than 1000 yds		- 100% inspection

    - Shipment 	1000 yds to 10000 yds	 - at least 1000yds inspection

    - Shipments  	more than 10000 yds 		- 10% of the lot.

4-Point system

The oldest inspection system is known as the “10 point system” in 1959, the natural association of shirt pajama sportswear manufacturer along with the American Society for Quality Control (ASQC) proposed “4-Point system” for fabric quality. Later on this system has been accepted by American Apparel manufacturer association, American society for testing and materials and department of defense USA.

Penalty Point Evaluation:-Fabric to be graded will be examined from a distance of approximately three feet while running force up over the perch of a fabric inspection machine. The inspection will run the fabric at a speed compatible with his ability to distinguish.

Point calculation:

- Individual Roll

Penalty points X 36 X 100.

Width of cloth in inch X Total yds inspected

= Points per 100sq.yds.

- Average Points per shipment

Total Points per 100 sq.yds of all rolls inspected

Total number of rolls inspected.

= Average points per 100 sq.yds.

The position of the defect in the fabric in either length or width

Defects Points Allotted

Up to 3”                                                                1 Point

Over 3” up to 6”                                                  2 Points

Over 6” up to 9”                                                  3 Points

Over 9”                                                                 4 Points

Any kinds of holes                                               4 Points

Acceptable penalty points

If no specified standard provided for woven fabric the average point value per shipment not exceeds 20 points per 100 square  yards. Individual rolls exceeding 28 points shall be rejected.

For knitted fabric the average point’s value per shipment does not exceeds 30 points per 100 sq. yds. Individual rolls exceeding 35 points shall be rejected.

## Defects conditions :

- No more than 4 points will be penalized within one linear yard.

- Severe defects are assigned the max 4-pointsfor each linear yard.

- Any hole and any running defect will be penalized 4 points.

- Whole roll running defect or continuous defects more than 10 yds will be rejecting per roll.

Splice Allowance :

- It will not be allowed more than one splice.

- No part of a spliced roll can shorter than 40 yds except specified by customer.

Roll Length :

- Roll found to deviate more than 2% from the roll ticket yds shall be rejected

- The entire shipment shall be rejected if the total measured yds in the sample audit deviates 1% on more from ticket yds.

Hand feel :

The hand feel will be checked between roll and to a reference sample. If there is noticeable difference, the roll will be classified as second quality.

Shade Matching :

Roll to roll and head to end, side to side, side center side will be checked during visual quality inspection for preliminary. Final checking will be taken in the light box against gray scale rating-Under light source D-65, TL-84

Color Shading Rating :

## Side to Side			4-5 or 5 (Accepted)

## Side center side        		4-5 or 5 (Accepted)

## End to End                		4-5 or 5 (Accepted)

## Lot to Lot 			4- or 4- 5 or 5 (Accepted)

## Against approval sample 	4 or 4-5 or 5 (Accepted)

5. Pre Production (PP) Meeting:

The meeting will be held based on Size Set evaluation report and the meeting will focus/covered the followings:

- Discussion about Critical path

- Quality issues as well as specification

- Bottle neck of production

- Store report

- Machine lay out

- Machine and manpower status

- Maintenance

- Production Target

- Input date

- Pattern

- Embroidery / Braid, print etc (If applicable)

- Washing standard, Recipe, Chemical, Temperature & Time cycle etc (if applicable).

6. Participants of the PP Meeting:

- Head of Quality or Responsible Quality Manager.

- Industrial Engineer & Planning responsible

- Concern Merchandiser

- Manager / Production Head

- Technical Head

- Manager Store / Store Responsibile

- Manager Cutting / Cutting  Responsibile

- Manager Laundry / Wash

- Quality in charge

- Line Chief

- Quality supervisor.

7. Pilot Run (PPR):

  - Inspecting the pilot run products for, visual, construction and measurement problems.

  - In case of any regular measurement problem, checking the patterns.

  - Informing Head of Quality about problems found in the pilot run product and suggesting solutions.

  - Discussing various complicacies, this can arise during production with Head of Quality.

8. Cutting Quality Management System:

- Fabric inspection

- 100% Fabric relaxation-24 HRS ( Maxim 03 Feet Height)

- Spreading control-Check the lay tension, Marker check, Lay count& Lay Height

- Lay chart and remnants yards maintain

- Measurement check report-Top, Middle and Bottom ply against the pattern

- 100% cut panels inspection

- Replacement report

- Sharp tools issues and return log report

Cutting Quality Control

The spreading of the fabric in cutting is audited by the Cutting Q.A. randomly for all the layering and marking problems like problems in table marking, marker placing, ends, splices, narrow goods, leaning, tension, counts, remnants, fabric flaws, etc. Report of the same is made and discussed. Refer Spreading Quality Control format.

Bundles of cut pieces are checked randomly by the Q.A. (cutting) as per auditing system for all the cutting defects like miss-cut, matching plies, ragged cutting, notches, etc. Report of the same is made and discussed. Refer to Cutting Quality Control format.

Q.A. Cutting :

  - Auditing the cutting as per the required AQL.

  - Keeping all the cutting department's personnel aware of the quality requirements.

  - Reporting to Head of Quality Area In-charge about the problems in cutting department and suggesting the solutions for the same.

  - Helping cutting department's personnel to implement new solutions and system.

  - Reporting to Head of Quality/Merchandising and Production about the problems related to fabrics and accessories.

Size Set Sample:

Before starting bulk production, a few pieces of garments (Cover all those size) are made in actual production conditions and system using actual fabrics. Investigations are made during making of the garment by the Production teamin assistance with the Q.A.&R&D team. The various aspects to be looked upon during Size Sets are as follows:

- Product specifications as well as fitting check.

- Styling and Threads-color, count/ply, quality etc.

- Workmanship as well as stitching-Machine Tension, SPI, Pressure etc.

- Embroidery, appliqué, Print etc (If applicable).

- Washing standard.

- Fabric Quality, like grain line, slit line, G.S.M., elasticity, whiteness, shrinkage, etc.

- Attachments, guides/Folder to be used in sewing machines.

- Construction problems and preventive action to be taken.

9.  Sewing Quality Management System:

- Pre-Production meeting

- Trial cutting/Size set report

- In-process QC report

- 7 Pieces Random inspection report

- End table Final inspection report

- Hourly audit report

- Measurement report twice daily (Before & after Lunch)

Sewing Quality Control:

Auditing is done in sewing at AQL 1.5systems. Seven pieces of a bundle are checked and if it fails as per standard AQL 1.5, other three bundles of the same operator are checked randomly. Refer to the Inline Audit Report. The various problems to be checked are listed in the list of the defects in annexure report.

Apart from this audit system after every critical operation checkpoints are established, with in a line of production, where 100% checking is done. Refer to the Inline Inspection Report.

A checkpoint is also established at the end of each production line and 100% checking is also done there. Refer to the End of Line Inspection Report.

Quality Control - (Sewing Section) :

  - Assisting Q.A. (Sewing) in his work.

  - Auditing the Sewing operation and stitched garments and reporting the problems to Head of Quality and others concern.

  - Discussing the style/buyer related quality requirements with Q.A. and communicating the same to supervisors/checkers/operators.

  - Other than above-mentioned individual responsibilities the Q.A. team also performs following functions:

  - Assisting R&D team for its work on forthcoming styles.

  - Making discussions and taking decisions on problems rose to suggest corrective/preventive action.

  - Maintaining the discipline and work environment in the garments production area and assisting the compliance department.

  - Fulfilling any other requirements, this got raised due to introduction of new styles/buyers.

Quality Assurance – (Sewing Section):

  - Auditing the Sewing operations and stitched garments as per the required AQL1.5

  - Keeping supervisors and checkers aware of quality requirements of style/buyer.

  - Reporting to Head of quality/Area In-charge about any problem (or Forthcoming problem) in Sewing department and suggesting the solutions for the same.

  - Helping Sewing department's personnel to implement new solutions and system.

  - Communicating with sewing operators about the requirements of a style/buyer.

  - Recommending area in-charge about line and system planning techniques to obtain better quality.

  - Measurement check twice daily.

10. Quality Control of Subcontractors / Suppliers (Printing, Embroidery) :

  - Discussing the job work requirements of a style with Head of Quality in advance.

  - Communicating the style requirements and quality standard requirements with the vendors doing job work.

  - Approving the samples produced by the various vendors in consultation with Head of Quality.

  - Visiting the vendors' premises to communicate the requirements to the individual job worker or in case of any quality related problem with the vendor.

  - Auditing the product delivered by the vendor as per the required AQL.

  - Consulting Head of Quality in case of any problem.

  - Maintaining quality related record of various vendors and analyzing it time to time for performance rating of vendors and deciding about the company's policy about a particular vendor.

  - All Embroidery products must have to pass through metal detection procedure.

Quality Control of Sub-contractors / Suppliers:

As and when necessary for all over print / Printing, Embroidery, Sticker etc.

The approved suppliers are contacted. It may be mentioned here that, these suppliers /factories have already been audited by the Compliance team of DIPTA Apparels Ltd. Based on the capabilities, Quality and price level and finally taking into consideration of the Audit report the suppliers are approved. Among the approved suppliers, and as per the requirement of the specific order, the work is assigned to them. The samples so collected from the suppliers are checked thoroughly and forwarded to the customers for their approval. Upon receipt of approval, the work orders are placed.

The field supervisors / executive / Quality Controller / Coordinators are sent to the respective supplier’s / sub-contractors premises to monitor the quality level .At the time of receiving the products after job work, the receiving person provides the following documents to Quality Assurance department:

- Challan

- Checking report of field supervisor

- Requisition for sampling inspection

All subcontract embroidery garments parts should have to pass through metal detection system.

11. Finishing Quality management:

- Process Inspection report

- Inline inspection report along with measurement

- Hourly audit report

- Two hours audit report

- Packing audit report

- Pre-Final Inspection report

Quality Assurance (Finishing section ) :

  - Auditing the finishing area on the hourly basis and reporting the problems found to the Head of Quality.

  - Maintaining the Quality System requirements in the finishing area.

  - Keeping the supervisors and checkers well aware of quality requirements of particulars style/buyer.

  - Discussing the style related problems with Head of quality and Q.A. (Final Audit).

  - Discussing about the problems in job work with Q.A. (outside job work).

  - Assisting Q.A. (Final audit) during final audit of the packed shipment.

  - Maintaining the system of checking and finishing as set by the Head of Quality/Q.A.

  - Auditing the finished product for measurement time to time.

Finishing Quality Control:

Each final checker sticks its checker code no for identification. Q.C. in the finishing audits the final garments randomly on daily basis and makes a report, which includes name and no of the checker found in the garment having problem. Refer to Finishing Audit Report. This report is used for timely analysis of the finishing quality and checkers' performances.

Apart from this measurement reports are also made by the Finishing Q.C. from time to time. The finishing Q.C. remains on the floor for all the time to maintain the system of finishing.

QA Pre-Final Audit:

      - Performing Pre-Final Inspection every P.O. (Shipment) of packed garments for visual quality, Packing and Specification as per the AQL Standard set by the company or as per buyer's standards (If specified).

      - In case of failure of any shipment, to discuss it with Head of Quality and to decide about the further course of action required for correcting the quality problem.

      - Deciding and informing area in-charges about the preventive action to be taken by them in next running orders of the same style, as the problems rose in the packed shipment during Pre-final audit.

      - Discussing the problems found in the packed shipment with Head of Quality.

      - Discussing the various ongoing problems with buying house Q.C. to find out solutions and quality standard requirements of buyer for a particular style.

Final / Visual Audit:

Every packed shipment is checked for its quality as per the required AQL standard as per the company policy or as per buyers' standards (if Specified). The various problems to be checked for are listed in the list of defects. Refer to the Visual Audit Report.

Packing Audit:

Every packed shipment is checked for ratio, color code, barcode, country code, carton marking, folding, packing material used, etc. as per the required AQL standard set by the company (annexure B) or as per buyer's standard (if specified). Refer to the Packing Audit Report

Apart of packing audit there is one more check point that PACKING/CARTON AUDIT REPORT

Carton Box Details: i.e.-Box quality, Number of ply, Thickness, Measurement/Size, Stickers quality/Glue.

Carton Stickers details: i.e.-PO #, Style #, Item #, Description, Color, UPC/Barcode, and Size/Quantity/Assortment.

Measurement Audit:

Every packed shipment is checked for measurements of the garments as per the required AQL standard as per the company policy or as per buyers' standards (if Specified). Refer to the Measurement Audit Report.

Quality Report :

Other than normal reports in each area of garment production, Quality Reports are also made for escalating the value of problems and to highlight them. The corrective and preventive actions for the problem are also recommended by the Q.A. and the comments of the area in-charge are taken for the approval of the corrective/preventive actions. The copies of these reports are sent to the concerned personnel/department and the same are filed in the style production file.

Statistical Analysis:

Statistical Analysis is done to improving quality and deciding about the problem area and causes of the major problems to eliminate them. Statistical Analysis is done at the following stages:

a.	Vendors' analysis in case of outside job work to get the overall performance of each vendor.

b.	Checkers' performance grading to make them competitive and motivated towards Quality.

c.	Final Audit analysis of that done by the in-house QA and buying house Q.C. to check the overall performance of the system and buyer's satisfaction level.

Style File:

For each and every style QA holds separate QA file, which contains followings:

  - PP Comments (Latest).

  - PO Sheets.

  - Updated Specifications.

  - Tech Pack along with sketches.

  - Approved Trim card.

  - Approved Fabric Swatches.

  - Bill of Materials (BOM/AOF).

  - Approved Shipping mark.

  - Packing instruction.

  - R&D report (to be provided by R&D Dept).Any others special requirement, if applicable.

Others Files:

Other than this style file (separate for each style) Q.A. department has hold of following files:

  - Spreading audit file.

  - Cutting audit file.

  - Sewing in-line audit file.

  - Finishing audit file (including finishing analysis report).

  - Embroidery/outside job work audit file (including job work analysis report).

  - Washing menu file.

  - Inspection requisition file (this includes the requisition given by production dept. time to time for final inspection).

## 12. Defect Codes Classification:

## DEFECT DEFINITIONS :

CRITICAL DEFECT: a defect likely to result in a hazardous or unsafe condition for an individual using the product or fails to meet mandatory regulations.

MAJOR DEFECT: a defect that is likely to result in failure, reduce the usability & salability and can easily be detected by the customer.

MINOR DEFECT: a defect that does not reduce the usability of the product, can be noticed only after careful examination, and may reduce the salability. A workmanship defect beyond the defined quality standard.

- The classification of a defect depends on the degree of severity.

- If any found defect is not mentioned in this inspection protocol, it will be classified according to above definition.

13. Standard Operating Procedure (SOP) of soiling control:

- Please cover the Needle Bar by knit fabric.

- Clean the Machine at least 01 in every 03 hours.

- Put the marking paper under the Pressure Fit during leaving.

- Next morning check out the things how much oil has bled in paper, if it is wide spread then call respective maintenance individual to take immediate action.

- Please block the oil Out Line of the machine.

- Should be well aware all the concern about House Keeping as well as make sure those products is not laying along with machineries.

- Reduce Oil level of the machine as much as below.

- Cleanliness: First of all please follow SOP of oil stain control as well as use Poly bag- Cutting to Finishing (Especially for color White/Light) to protecting the products from Oil/Soil mark & please try to keep neat and clean the production floors make sure the products are not laying on the naked ground, also need to improve housekeeping of the overall plant.

14. Section /Processwise Quality Policy & Procedure

## Metal detection policy:

- A METAL DETECTION AUDIT IS ONLY MANDATORY FOR CHILDREN’S, INFANT AND TODDLER PRODUCT.

- METAL DETECTION OF ALL FINISHED PACKED PRODUCTS IS A REQUIREMENT. METAL DETECTION MINIMIZES THE RISK OF METAL CONTAMINATION PRESENT IN THE PRODUCT AND IS SUPPLEMENTARY TO THE FOREIGN OBJECT CONTROL POLICY AND PROCEDURES.

- Total quality of each baby and children order should pass the metal detector machine.

- If any sample fails, find the metal part and keep record it in a separate report.

- If metal part is not found put failing garments side & make sure it is destroyed.

- Each garment should pass the metal detector one by one as a final check just before packing.

- Make sure garment does not return back in to production unit after metal detection is made.

- Metal detector should be calibrated at least three times daily.

For any garment failing the metal detection test, the inspection will have to be considered as a fail.

## 15. Procedure for children’s Products:

## All the time of PP meeting risk assessment sheet must be attached with PP meeting format.

- All critical measurement must be check at the time of Size Set making.

- All production area must be metal free zone, especially finishing & packing.

- Needle control log book must be maintained, broken & disposal needle record to be maintained.

- Chemical should not be used for spot removing purpose.

- Neck stretch should be check by neck gauge

- Button attachment strength test & recorded.

- Snap attachment strength test & pinch setting & recorded.

- Snap attachment check visual/Physical.

- All other product safety points as per style require being check.

## 16. Fabric Relaxation policy:

Stretch Fabric:

- In order to avoid Measurement issue of the products-all stretch has to be in relaxation minimum 24 hours prior to spreading & relaxation height not more than 03 feet.

- After spreading has to keep more one hour to get proper result.

Regular Fabric:

- In order to avoid above issue all regular fabric should be keep in relax one hour after spreading.

Note :

- Fabric cannot be kept on tide condition.

- Standard height of relax fabric will have to maintain not more than 03 feet.

- Should have to keep in cover

- Not keep mixing with light & deep color together.

- Should have product identification.

## 17. Policy for Popper, Rivet, Press Stud & Button attachment:

- All poppers, Rivet, Press stud should be purchase from nominated supplier.

- There should be a technical data sheet on m/c and production should be as per the same.

- M/C Mould/Dais should be from nominated supplier, who is supplied Popper, Rivet & Press Stud.

- Snap attachment strength test should be check daily basis & have to arrange mat under snap button machine to stor spread out rivet/button etc

- M/C Pinch setting should be check daily basis.

- Snap attachment should be check Visual/Physically daily basis.

- Button should be attaching by Lock Stitch m/c.

- Button attachment strength test should be check as per buyer requirement daily basis.

- Every new bobbin filling first attached button should ne check.

- No rabbit ear should be cut.

## 18. Operating procedure of Fusing Machine:

In order to control fabric shrinkage and other problems like peelings, Bubbles etc. occurred due to fusing process to certain garments components ,following SOP (Standard Operating Procedures) is used must be followed.

Fusing-Study of Fusing Shrinkage:

- Cut the fabric swatches with squire template of size 18”x 18” (or 12”x12”)

- Fused the fabric & specified interlining as specified temperature, presser and time.

- Allow for cooling of the fused parts.

- Check for fusing shrinkage and record same…incorporate shrinkage into the patterns “As allowance for fusing”.

- Check for shade variation if any due to fusing, if you observed any variation, and then report the cutting manager as well as respective person and initiative to correct the things.

- Check the fusing for defects like-peel off, Bubbles etc. inform according to above to get the solution.

- QA has to be educated/specify area of fusing and temperature and pressure to be set for fusing

- Fusing tests must be carried out twice daily, once at the starting of the m/c then again in the afternoon

Fusing machine-Tests & daily maintenance:

- Fusing validation trial tests must be carried out regularly, when the new style begins fusing parameters has to be finalized (temperature, time & pressure).

- Check the fusing machine pressure rollers nip, affecting the fusing quality, by passing a fabric strip through the machine & by pulling it backward-the need for re-alignment of the rollers pressure can be identified when this is done at three points i.e. left centre/right of the belt.

- By passing a carbon paper placed on top a white paper through the machine the impression on the white paper can be observed. This would be also will help us to determine any inconsistence of the roller pressure at left, centre or right.

- Cleaning of the belt thoroughly and check for the required temperature & pressure when the fabric/color changes.

19. Complaint / Feedback Handling Procedure

If we received any complain from customer firstly we read it carefully and review Complain details and take necessary correction for specific order.

After that we arrange a meeting with concern department / section and do root cause analysis for identifying real cause. Based on root cause analysis we make corrective action plan and implement accordingly. Also make a list those management person will follow up this complain.

And assign specific duties & responsibilities to make corrective the complain.

Finally we send approved corrective action plan to the customer. Management meeting is arranged regarding this issue.

Complain Handling Record Form (Sample)

Date:

Complain Receive Details:

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………….

Correction:

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Root Cause Analysis:

…………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Corrective Action Plan:

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Corrective Action plan Implementation:

……………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

Follow Up By the Management:

………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………………

DGM HR &Compliance 			GM(Production)		H/O(Quality)

20. Product Recall / Withdrawal Procedure

Once we received any complain from customer firstly we read it carefully and review Complain details and take necessary correction for specific order.

In case of recall / withdrawal during finished goods warehouse or after shifted the product, we will arrange a management meeting and will communicate with buyer about complain. At first we will try to negotiate the matter as per demand of buyer and if it is not possible, product will do withdraw. After withdrawal, we will arrange special meeting with concern responsible and will analyze the root cause and also will take necessary action to rectify the problem (if it is rectifiable).

But if the defect is non rectifiable, then the entire brand label will remove from the entire product and will dispose the product as per company policy.

Finally we send approved corrective action plan to the customer.

21. PROCEDURE FOR PRODUCT CLAIMS

- Whenever product claims like Teflon coated, Organic cotton and performance finished such as anti-bacterial will run in unit the area should be marked with colour tape or paint for different identification.

- In fabric store & checking the area to be marked with colour tape or rack to be different colour.

- In cutting where this fabric will cut the cutting table should be cleaned & cut panel should be tied with same fabric cord & kept separated with identified area.

- In stitching the line should be clean (M/C, Table) and identified with colour tape. Also covered around with fabric or poly sheet to avoid different fabric dust.

- The garment movement to finishing in different marked   source.

- In finishing pressing table should be covered with same fabric & identified with colour tape.

- Checking table should be clean before start garment & it should separate from other products and also be identified with colour tape.

- Packing area it should be separated & marked with colour tape.

- All documents should be highlighted with colour tape or different colour page.

22. PROCEDURE  FOR LABEL MANAGEMENT ( SEWING )

- Labels should be stored with production supervisor in secure place.

- Labels must be issue to lines as per cutting chart with same size, quantity & colour.

- Operators inserting label as per bundle wise & it should be recorded.

- Replacement labels issue due to loss, damage or incorrect will be issued as on recorded basis.

- All spare labels issued to sewing line will be returned to line supervisor.

- End of order all remaining labels will returned to accessories store.

23.  PROCEDURE FOR TRIMS /ACCESSORY INSPECTIO,HOUSE KEEPING & SYSTEM

- When any new batch comes it should first kept on quarantine Area for inspection.

- Approved trims cards should be present at the inspection area.

- If any trims rejected it will sent to non-conforming area & ok goods sent there appropriate place.

- All area of store having sufficient space, good light & cleanness all.

- Check batch for colour fastness.

- Check all trims for Quantity, quality & size.

- In case of multiple trims minimum 10 units to be check from each box.

- Making all records for inspection.

24. Procedure for Children’s Products

- At the Time of P.P.M risk Assessment Sheet must be attached with P.P.M Format.

- All Critical measurement must be check at the time of size set making.

- All production area must be metal free zone, especially finishing & packing

- Needle control log book must be maintained, Broken & disposal needle record to be maintained.

- Neck stretch should be check by neck gauge.

- Button attachment strength test & recorded.

- Snap attachment strength test & pinch setting & recorded.

- Snap attachment check visual / physical.

- No chemical used for spot lifting purpose

- All other product safety points as per style require to be checked.

25. PROCEDURE FOR HAND SEWING NEEDLE & TOOLS CONTROLS

- Any established quantity of hand sewing kits (This could include needle, scissor, clipper & cutting pins) must kept by authorized person in safe place

- The kit must be issued to each operation on record “one in one out “as log book Daily basis

- The designated or quarantine area must be established for keeping the tools.

- All products which are having hand needle work have to be passed through needle detector m/c before to next process.

- All above should be recorded in a manner way.

26. PROCEDURE FOR KNIFE BLADE CONTROLS

- This procedure is maintained in cutting department where the straight or band knife m/c working.

- The all blade should be kept or replace by authorized person.

- The policy of issuing blade is “new for old “basis in a controlled and easily auditable manner in a safe place or quarantined area.

All broken parts of any blade to be summated than only new blade should be issued and it should be recorded.

27. PROCEDURE FOR LABEL MANAGEMENT IN WAREHOUSE

- All labels & hangtags must be controlled and placed on quarantine area until it checked & approved.

- The labels & hangtags should be checked as per approval trim card. It must be checked at least correct country of origin , Fiber composition , care code & Product number .

- All labels & Hangtags should be issue to production section by line wise feeding plan.

- Labels & hangtags must be kept style wise with bin card.

- In store there should be stock control system maintained.

- When Spare label & hangtags returned from production floor it should be checked & count and kept in there appropriate place.

- All label management records to be maintained.

28. PROCEDURE FOR MEASUREMENT AUDIT

- Approved measurement sheet must be available at the place of audit.

- The measurement audit must include the full size chart dimensions.

- When any measurement found out of tolerances it should highlighted & whatever corrective action taken it should be documented.

- Any garments is rejected it should be kept separated & also recorded.

- When the measurement audit is been for children product there should be more attention on critical points which is identified in risk assessment.

- All above works should be well documented.

29. PROCEDURE FOR POPPER, RIVET, PRESS STUD & BUTTON ATTACHMENT

- All popper, rivet & press stud should be purchase from nominated supplier.

- There should be a Technical Data Sheet on m/c & production should be as per same

- M/C Dye should be from nominated supplier, who is supply popper, rivet & press stud.

- Snap Attachment Strength test should be check daily basis.

- M/C Pinch setting should be check daily basis.

- Snap attachment should be check visual / physical daily basis.

- Button should be attaching from lock stitch m/c.

- Button attachment strength test should be check as per buyer requirement daily basis.

- Every new bobbin filling first att. Button should be check.

- No rabbit ear should be cut.

30. PROCEDURE FOR PRE-SHIPMENT AQL

- Before offering the final goods to external Q.A it should be pre audit by internal Q.A.

- The pre –audit should be done as per required AQL process.

- 100% of finished goods go through the AQL process.

- All approved samples & trims should be available in the audit place.

- The inspection process includes measurement, stitching, fabric & accessory quality against approved standard.

- All rejected batches should again go through 100% checking.

- All 100% re-checked batches go through AQL.

- For children product AQL must check all safety issue identified in risk assessment.

31. PROCEDURE FOR RAW MATERIAL APPROVAL

- Shade should be matched with approval lot. If there is any discrepancy so manager will take commercial decision with help of buying house team.

- G.S.M should be within +/- 5%, If there is any discrepancy that also be taken decision as above mention point.

- If lot is having problem as per 4 point system, & taken commercially decision by management.

- If there is any above point happened it should be documented with authority signature.

32. PROCEDURE FOR RAW MATERIAL INSPECTION & TESTING

- Minimum 10% rolls to be picked from lot for audit.

- Fabric audit should be done on 4 point system.

- All rolls shade continuity should be maintained.

- Fabric inspection machine should be placed in suitable place & there light should be 1000 lux.

- Every lot should check for shrinkage, GSM, spirality.

- Make mock for checking trims shade & fabric rapture test.

- All records should be maintained.

33. PROCEDURE FOR RAW MATERIAL CONTROL

For product safety issue Raw material / accessories control is very important aspect. In absence of maintaining proper controlling mechanism overall production may hamper & also cause problem as follows ;

Mixing with another product / Brand : In absence of proper identification or controlling mechanism of raw material it is very much possibility to mix with other product or item.

Probability for lost: Not for controlling raw material as per standard it has become possibility to loss in bulk production.

For controlling Raw Material Control following procedure should have to done;

- There should be separate marked area for inspection waiting goods.

- After inspection the material should move to their nominated & identified racks.

- All parameter should be checked & documented as per buyer requirement.

- Material should be kept with all information.

- If any material will be pending for decision so it should be store in non confirmatory area.

- Inspection to be done by trained staff or workers.

All procedure for checking should have to keep in record.

34. PROCEDURE FOR USED NEEDLE CONTROL

- This Procedure for all type of needle used in production floor.

- When a used or bland needle needs replacement it should be replaced by authorized person from the controlled stocks.

- New needle can only issue behalf of returned needle as same type or quality of needle.

- The exchange used needle must be kept in control manner in secure place.

- All above records should be maintained & kept minimum one year.

- Exchange needle should be kept on “Broken/Used Needle Record Forms”

35. Guideline of Mold Issue

Due to controlling mold in garments additional procedures and check points implemented in DIPTA Apparels Limited.

To avoid entirely mold development and spreading inside manufacturing facilities & also in transport of goods throughout the production process, including transport of finished, packed goods to Chittagong below precaution measures should have to take.

In-Factory:

- Have to install humidity meter in all Production Areas including Laundry. must not exceed 60% level. If it does, circulate and treat air as necessary to drop to below 60%.

- Have to install portable Moisture level meter to test garments in random

- In Laundry - after drying.

- In finishing, last step, before poly-bagging & carton

- Have to install and press our goods only on vacuum press type tables usage

- In Laundry - verify Extracting is performed in full cycle, sufficient time, for maximum water extraction.

- In Laundry – make sure sufficient drying time at suitable temperature to dry garments completely.

- Every six (6) months, all factory facilities (floorings, walls & ceilings) need to be decontaminated for mold using bleach water solution. Log and record all such treatments.

Transport :

a.      Trucks and Loading:

  - Use only trucks with full steel cover van.

  - Ascertain the steel box/van has no holes or spaces that permit any water in.

  - Ascertain before loading the steel box/van has is clean, dry and has no bad odor inside.

  - Humidity level inside box / van should not exceed 60%.

b.      Loading Area:

  - Loading must be done under roof – when entire length of truck is covered, fully protected from rain and water drops.

  - Make sure all cartons are packed and sealed well, must not be broken

  - Make sure no wet cartons are loaded.

Wet or broken cartons will be rejected shipment at forwarder’s facility and you will be asked to collect them back**

Photo Documentation – take pictures and record, for every truck used for transport:

  - The plate number

  - Inside condition - flooring, walls, top roof & main door

  - Humidity meter inside used recording the humidity level

  - Loading area with full length roof top covering the truck during loading of cartons.

  - Truck Cover Van Door Seal Number

Document Proofs Dissemination:

  - Hand-over to truck driver the Chalan with the complete information including :truck plate number; truck driver & helper names; condition inside box/van; weather condition during loading (rainy or clear), date & time of departure from factiory with signature & seal

36 Product Withdrawal & Recall Procedure

Introduction

In the event that a Garments safety issue arises with our products of DIPTA Apparels Limited will protect public health by facilitating the efficient, rapid identification and removal of unsafe product from the distribution chain and, by informing Customer (where necessary) of the presence in the market of a potentially hazardous product.

There is a documented recall procedure in place and this will be periodically tested to ensure that it is Comprehensive and fit for purpose in its ability to remove an unsafe product from consumers and/or the distribution / Supply chain.

Recall Procedure

This procedure states the actions of DIPTA APPARELS  LIMITED will take to effectively manage the recall of a product which has been determined to be unsafe or unsuitable.

There are two levels of product recall, these are as follows;

1. Recall (also known as a consumer level recall): This is a removal of unsafe product from the distribution chain/buyer and extends to product sold to consumers and therefore involves Communication with consumers.

2. Withdrawal (also known as a trade level recall): This is the removal of an unsafe product from the distribution chain but does not extend to product sold to the consumer.

In the event of product withdrawal following steps are taken:

- Goods are counted and registered.

- Label is removed.

- Goods are quarantined.

- Customer is notified of action taken.

- Packing list is made accordingly.

- Authorization is requested from the customer if goods can be.

- Sold as stock lot after quarantine time has passed.

- Goods are destroyed (brand dependent).

If goods have been agreed to be destroyed, photo evidence is submitted to the customer.

Roles and Responsibilities

It is DIPTA APPARELS LIMITED responsibility to effectively organize and manage the recall of our product that has been demonstrated to be unsafe or unsuitable.When a recall is initiated, our actions in recalling the affected products need to be co-ordinate with the Buyers. We shall notify Buyers as soon as a recall is likely. It is our responsibility to manage the recall by clarifying the product safety issue and the exposure (who and where risk exists), and to provide details on distribution and the method of recall.

The Recall Committee

The recall co-coordinator Head of Operation will initiate the formation of a committee and will co-ordinate actions with Buyer.

Committee members will include personnel from across DIPTA/DIRD  GARMENTS LIMITED & DIRD Washing Plant Ltd. Typically the committee would have a mix of knowledge across the following areas;

- Head of Operation

- Head of Quality

- Head of Merchandising

- Production Head

- Commercial Department

The recall committee is responsible for the management of all recall activities and to adhere to this procedure. Duties of the recall committee are to:

- Assess the overall problem;

- Notify the relevant regulatory authority;

- valuate the hazard in the product and the extent of contamination;

- Determine a strategy to be followed;

- Make decisions about product still in manufacture or in storage;

- Decide who makes any press statements;

Recall Actions & Documentation

The recall committee shall reference and follow the actions outlined in the Recall Guidance Material when we become aware a product may be unsafe or unsuitable. We will ensure that records of all actions and decisions and who was responsible are recorded and retained.

Decision to Recall

The decision on whether to recall or withdraw a product/s or not will be based on the identification of a hazard that makes a product unsafe and its likelihood of affecting public health. This will be determined by careful, considered risk assessment. The recall committee will conduct a risk assessment using the Hazard/Risk Assessment and we will include the appropriate regulatory authority in the process. We will refer to the Recall Guidance Material on the roles of regulatory authorities in regards to a recall.

Scope of Recall

The scope of a recall is a very important part of the process; it ultimately ensures the effective identification of all affected product/s, ingredient/s and location/s. We will follow the requirements set out in the Recall Guidance Material to ensure our plan incorporates the details mentioned.

Notification of a product recall

If the decision is taken to initiate a Withdrawal we will notify:

- Senior management

- Buyer/Customer

- Anyone that has received our product, including distributors, wholesalers, retailers.If we are engaged in a Withdrawal but find that for whatever reason that it is not possible to contact all relevant customer then we will consider expanding the Withdrawal to a Recall. If the decision is taken to initiate a Recall we will notify:

- All people mentioned under initiation of a withdrawal, outlined above and The contact list must contain the contact details for the following:

- The product recalls committee and senior management and key company personnel.

- Suppliers of all ingredients.

- Distribution company and business customers.

- Sources of technical advice and support including laboratory facilities.

- Regulatory authorities.

Communication

Notification in respect to the recall needs to be done promptly and should cover the following areas;

Regulatory Authority

We will notify the appropriate regulator at the earliest opportunity, after an incident is identified that may lead to a recall. We will supply as much information as possible, using the Hazard/Risk Assessment and the Recall Guidance Material. The regulatory authority will be updated throughout the process.

Regaining control of affected stock

If affected stock is directed to be returned to us then the recovered product/s will be stored in an area that is separated from any other products. Accurate records will be kept of the amounts recovered and the codes of the product/s. If the recovered product/s is unfit for human use, it may be destroyed or denatured under the supervision of the company management and/or the regulatory authority where legally required.

Effectiveness of the Recall

To be effective, the product recall notification must reach as far as the product has been distributed. The effectiveness of the product recall is assessed on the basis of the amount of product returned as a proportion of the amount of product that left DIPTA Apparels Limited while taking into account time in the distribution chain and the retail turnover of the product. Progress of the product recall must be reviewed so that its success can be monitored. If it is decided that there is now little risk to the public, the product recall can be judged to have been a success and brought to an end, however if there have been few returns and little response to a high risk problem the product recall procedure must be reassessed. The product recall may have to be repeated using different methods to reach the customer.

Testing & Reviewing the Product Recall Plan

The recall committee will review this procedure every twelve months, and the contact list will be amended as required. The procedure will also be reviewed after any recall, the review will consider the elements suggested by the Recall Guidance Material.

We will conduct a mock recall exercise within three months of the initial development of this procedure and additional mock recalls will be conducted on an annual basis. Records of these mock recalls will be documented and filed in the DIPTA Apparels Limited. Once the mock recall is completed, a review must be carried out with the relevant recall committee members to correct and improve the process where necessary.

Recall Report

We will submit a recall report to the regulatory authorities within an agreed timeframe of the closure of the recall. The final report will include the elements outlined in the Recall Guidance Material.

37. Raw Material Supplier Approval/Selection Procedure

Introduction:

An organization may use any criteria arising from the organization’s core processes requirements. Standard approval criteria generally intend to cover issues such as quality, capacity in terms of finance, services, and equipments, quantity, responsiveness, and others. Dird Group approval is required for all who supply raw materials or services. All written and oral communications with the supplier as well as the customer’s specifications, procedures and reports shall be in understandable language.

Approval/Selection Procedure:

- Supplier must have a quality management system that is compliant to  Quality Management Systems / Requirements.

- Dird Group shall assure that the material is distributed by sources approved by Dird Group’s customer.

- Raw material test reports and/or notarized reports, in accordance with all applicable specifications, must be kept on file and traceable to end item products delivered to Dird Group.

- Certificate of compliance with actual results of physical and chemical test reports must accompany each shipment where supplier furnishes the raw material.

- Test reports must be traceable to the material lots supplied.

- Supplier should contact concern of Dird Group for assistance in assuring the source of raw material supply is acceptable to customers of Dird Group.

- The raw material suppliers guarantees the right of access to their facilities and quality related data to Dird Group authorities.

- The supplier shall establish a system to retain records for a minimum of three years after providing service.

- Suppliers of raw shall retain any inspection record and delivery records, for the duration of the contract years.

- The supplier shall provide delivery documentation for each item delivered in accordance with the purchase order/contract.

38. Raw Material Supplier Performance Monitoring Procedure

Dird Group evaluates supplier performance in the areas of quality and delivery as applicable.

The supplier is responsible for complying with quality system requirements noted herein and for meeting performance expectations. Failure to comply with requirements or to achieve an acceptable performance level may result in an on-site audit and/or disapproval from Dird Group approved supplier list. Dird Group suppliers are required to maintain a Quality and Delivery rating of 98% or greater; failure to comply may result in the supplier presenting a corrective action recovery plan.

Delivery ratings are determined on the occurrence of the following criteria only when it is determined to be the Supplier’s responsibility:

| A | Late deliveries (Item(s) received after due date) | RED |
| --- | --- | --- |
| B | Early Shipment (Greater than 5 days) | YELLOW |
| C | On time (Between 4 days early and 0 day late) | GREEN |

Quality Ratings: This is monitored but not limited to a quarterly review through the NCR Reports.

| RED | <94.9 |
| --- | --- |
| YELLOW | 95.0-97.9 |
| GREEN | 98.0-100 |

39. Risk Assessment for Children Product

Objective

DIRD GROUP has a reputation for providing better & qualitative services to all supplier & brand. Also committed for qualitative production as per requirement of brand / buyer. For ensuring better quality production as well as good services DIRD group has a strong monitoring team. In DIRD Group (Savar Zone) DIPTA Apparels Limited  readymade garments factory produce & process garments for men, women, child, infant etc, Woven bottom & Knit product. And has a special care for children product in aspects of heath safety & product safety. Before design the product, sample development & bulk production the production team has conducted a Risk Assessment. In this Risk Assessment they conduct identify probable risk, responsible department / section & action plan. For risk assessment action plan following step & procedure has taken into consideration.

Scope

The policy covering DIPTA Apparels Limited. The requirements of this guideline apply to Children’s product design, Sample development & bulk production process. The guidelines provide a safety assessment framework that focuses on the known hazards, particularly relating to inhalation and ingestion, strangulation as well as other potential physical injuries.

Assessment of safety requirements applies initially to children’s product during design and development phases. The assessment must not only consider hazards that occur during normal conditions of use, but also hazards arising from reasonably foreseeable conditions of misuse or abuse.

The requirements of this guide specify acceptable criteria and in some cases manufacturing “best practice” for children’s product such as;

- Needle controlling activities during production process.

- Controlling Sharp tools during production process.

- Metal Trims, Rivets & Snaps.

- Embroidery (Both for Woven & Knit Product).

- Using Chemical for Washing Purpose (if required).

- Using chemical for spot removing purpose.

- Special Care during Packaging.

Consequence” relates to the severity when an event causes injury or damage. For the purposes of these guidelines, “consequence” is categorized according to the following table.

| Catastrophic | Consequence (for an injury) / Injury Outcome & Treatment |
| --- | --- |
| Major | Permanent Disability or Death. |
| Moderate | Extensive injuries requiring hospitalization or substantial treatment by a registered physician. |
| Minor | Injuries requiring minor treatment by a registered physician but not requiring hospitalization First Aid Treatment. |
| Insignificant | No Injuries. Mild discomfort or irritation |

The “Recommended Actions” are then determined based upon the risk rating. The higher the risk rating, the greater the effort required to alter the design and to re-engineer the product to minimise or eliminate the identified risk.

| High Risk | To mitigate the known/anticipated HIGH risks and to achieve a position of “tolerable risk”, actions should include; - Eliminate/remove the identified hazard, - Modify, re-design or re-engineer the product, - Introduce cautionary labeling & warnings (if appropriate) - Do not proceed with development of the product |
| --- | --- |
| Medium Risk | To mitigate the known/anticipated MEDIUM risks and to achieve a position of “tolerable risk”, actions should include; - Eliminate/remove the identified hazard. - Modify, re-design or re-engineer the product. - Introduce cautionary labeling & warnings (if appropriate). |
| Low Risk | To mitigate the known/anticipated LOW risks and to achieve a position of “tolerable risk”, actions should include; - Modify, re-design or re-engineer the product. - Introduce cautionary labeling & warnings (if appropriate). |

Minimizing Production Risks

The primary purpose of conducting risk assessments during a product’s design and development phases is to incorporate safety into the product from its origins. This is the single most effective strategy for eliminating product safety risks.

It should also be noted that, regardless of design, safety hazards may also arise within the production process. Risk may arise through:

- Contamination from equipment (e.g. broken needles)

- Self-contamination (e.g. buttons or press-studs loose in garments)

- Deviation from specification (e.g. trims not attached securely)

- Raw materials variability (e.g. fabric properties differ from approved fabric)

The use of production and post-production quality checks are required to ensure that quality and safety are not only engineered into products from the outset, but to ensure that safety transcends through the entire end-to-end process. Production and post-production quality checks may include the following;

- Audits of factories to establish quality capabilities.

- Quality inspections of manufactured goods to confirm items are as-approved.

- Quality & safety checks of goods as they are being produced (i.e. in the production line)

- Laboratory testing to validate that production items meet specified requirements.

- Production samples being submitted to the retailer/importer for approval prior to shipping.

There are 4 main classes of hazards associated with children’s apparel and accessories, Choking & Ingestion hazards

- Sharp Edges and Points hazards

- Strangulation and Entrapment hazards

- Chemical Toxicity hazards

- Other Sources of Hazards

Product Features

A risk assessment template has been created for each of the features commonly associated with fabric and accessory products:

- Embroidery

- Fabric Loops, Hanger Loops

- Buttons

- Heat-sealed Decorations

- Functional Ties, Decorative Ties & Sashes

- Metal Trims, Rivets & Snaps

- Zips - Slide Fasteners & Pullers.

- Packaging

Probable Sources of safety Risk / Hazards:

Injury by Sharp Edge :

Cause hart by sharp edge if available at the time of using cloth, The probable source / cause of sharp edge be from button, snap.

Injury by Sharp Tools / Needle :

In bulk production process if used sharp tools / Needle not control properly may contaminate to garments & cause for injury for children.

Burn hazards:

Hazards with the potential to cause injury by heat, chemicals radiation. Flammability hazards for children’s nightwear are covered by a mandatory standard. Sun protective fabrics can be tested and assessed according to the standard.

Insertion hazards:

Hazards arising from children placing small objects into body openings (e.g. ears, nose, but not the mouth). Small parts create the risk, similar to choking and ingestion hazards.

Pinching hazards:

Hazards arising from closing mechanisms that can pinch or squeeze the skin.

Projection hazards:

Hazards arising from pieces or components of garments/products which protrude in a way that could cause them to push-into or penetrate the body.

Tripping hazards:

Hazards arising from parts of garments that can float/sit on or near the feet (e.g. long belts on robes, draw-cords at/near ankles on pants)

Special Instruction for Remediation of Probable risk are as following;

- All the time of PP meeting risk assessment sheet must be attached with PP meeting format.

- All critical measurement must be check at the time of Size Set making.

- All production area must be metal free zone, especially finishing & packing.

- Needle control log book must be maintained, broken & disposal needle record to be maintained.

- Neck stretch should be check by neck gauge.

- Button attachment strength test & recorded.

- Snap attachment strength test & pinch setting & recorded.

- Snap attachment check visual/Physical.

- All other product safety points as per style require being check.

- No Chemical be used in spot removing for children product.

Conclusion :

Considering the above probable risk all factories of DIRD Group – Savar Zone for children product conduct Risk Assessment for all process of production as Product development, Bulk Production ie Fabric Receiving, Storing Fabric & Accessories, Bulk Cutting, Processing for Sewing, Embroidery (if needed), all process for washing (if needed) all process of dying (if needed) all process of finishing & packaging etc. And communicate to all for production process department to remediate the probable risk & needful action for this for ensuring better safety services for children product.

40. Warehouse Cleaning Procedures

Dipta  Apparels Ltd aware of the importance of the appearance of its business premises include warehouse space. It is important to keep the warehouse clean and tidy which could provide the business with a real boost.

- Storage:

  - All trims, accessories, components and packaging remain in a good and clean condition

  - Clear Segregation on acceptable and non-conforming materials.

- Sweeping:

  - Remove trash

  - Remove large debris from the floor

  - Remove walk-off mats

  - Use putty knife to remove anything stuck to the floor

  - Dust mop on an angle away from the wall

  - Work in ‘S’ pattern

  - Use dust pan and corn broom to remove sweepings

- Auto scrubbing:

  - Place "WET FLOOR" signs wherever necessary

  - Dispense cleaner into automatic scrubber following label instructions

  - Apply cleaner, scrub and pick up in one operation

  - Mop the corners and rinse the equipment

- Wet Mopping:

  - Set out "WET FLOOR" sign

  - Remove debris, walk-off mats and any objects stuck on the floor

  - Dust mop

  - Dispense floor cleaner into bucket, mix half-and-half with cold water

  - Dip mop and wring

  - Mop in a ‘figure 8’ pattern with overlaps

  - Rinse mop when both sides are soiled

  - Repeat the described procedure until floor is completely clean

41. wi‡R± wWm‡cvR bxwZgvjv Ges c×wZ (Recetction Disposal Policy & Procedure) t

- KviLvbvi ‡Kvb cÖWv± Gi Mvg‡©›Um, †j‡ej Ges Ab¨ †h †Kvb G·mwim wi‡R± n‡j cÖ_‡g mswkøó †mKkb D³ wi‡R± Gi aiY, wi‡R± Gi cÖKvi,  Ges evqvi Abyhvqx GKwU ZvwjKv ˆZwi Ki‡eb |

- D³ wi‡RKkb ZvwjKv Abyhvqx wWm‡cvR Kivi Rb¨ mswkøó †mKkb cÖavb mn †÷vi Ges GWwgb cÖav‡bi m¤§wZ ¯^vÿi wb‡Z n‡e Ges wWm‡cvR Gi aiY wb×viY Ki‡Z n‡e ( Av¸‡b cyov‡bv / wmRvi Øviv †K‡U bó K‡i †djv)|

- ZvwjKv Abyhvqx mKj cÖKvi wi‡RKkb AvB‡Ug¸‡jv KviLvbvi evDÛvixi wfZ‡i A_ev G‡m¤^jx c‡q›U G wb‡Z n‡e|

- D³ wWm‡cvR Kvh©µ‡g Aek¨B GWwgb, †÷vi, wmwKDwiwU †mKkb Gi cÖwZwbwa Dcw¯’Z _vK‡Z n‡e|

- mKj wWm‡cvR c×wZi Aek¨B †iKW© ivL‡Z n‡e|

- ‡Kvb AWv©i Gi wkc‡g›U †kl ev †K¬vR nevi Av‡M wWm‡cvR Kiv hv‡e bv|

42. Needle & Sharp Tools Control Policy:

wb‡Wj wbqš¿b bxwZt

- d¨v±ixi mieivnK…Z wb‡Wj wbqš¿b Kivi Rb¨ GK ev GKvwaK (cÖ‡qvRb n‡j) `vwqZ¡evb e¨w³ wb‡qvwRZ _vK‡eb|

- wb‡qvwRZ e¨w³ Aek¨B cÖ‡qvRb Abyhvqx wba©vwiZ cwigvY wb‡Wj msi¶b Ki‡eb|

- mKj wb‡Wj Aek¨B GKwU msiw¶Z I myiw¶Z RvqMvq ivL‡Z n‡e †hLv‡b ïaygvÎ Aby‡gvw`Z e¨w³eM© †h‡Z cvi‡eb|

- mKj myBs wb‡Wj †KejgvÎ †gKvwbK A_ev Aby‡gvw`Z e¨w³ Øviv cwieZ©b Ki‡Z n‡e| †Kvbµ‡gB Acv‡iUi wb‡Wj cwieZ©b Ki‡Z cvi‡e bv|

- myBs †gwkb Acv‡iUiiv †Kvbµ‡gB cÖ‡qvR‡bi AwZwi³ wKsev †Kvb fv½v wb‡W‡ji Ask wb‡Ri Kv‡Q ivL‡Z cvi‡e bv|

- wb‡Wj msi¶K bZzb wb‡Wj ZLbB Bmy¨ Ki‡Z cvi‡eb hLb mg msL¨K I mg mvB‡Ri e¨eüZ A_ev fv½v wb‡Wj †dir cv‡eb|

- bZzb wb‡Wj Bmy¨ Kivi mgq wb‡Wj msi¶K‡K Aek¨B wb‡Wj MÖnxZvi bvg, AvB wW, ‡mKkb, ¯^v¶i, wb‡W‡ji aib BZ¨vw` mwVK fv‡e cyiY K‡i †iwR÷vi †gBb‡UBb Ki‡Z n‡e|

- wb‡Wj msi¶K w`b †k‡l cÖ‡Z¨K cÖKvi wb‡W‡ji Rgv I Li‡Pi e¨v‡jÝ wgj K‡i ivL‡eb|

- ‡h mg¯Í †gwkb AweiZ e¨envi n‡”Q †mBme †gwk‡bi wb‡Wj wbqwgZ weiwZ‡Z cwieZ©b Ki‡Z n‡e, hv wb‡W‡ji ¶wZ Ges wb‡Wj fv½v Gov‡Z mvnvh¨ Ki‡e|

- ‡hmg¯Í myBs †gwkb Ajm fv‡e i‡q‡Q †mBme ‡gwkb n‡Z Aek¨B wb‡Wj Ly‡j wb‡Wj msi¶‡Ki wbKU Rgv ivL‡Z n‡e|

- †d¬vi n‡Z msMÖnxZ mKj e¨envh© wb‡Wj msiw¶Z ¯’v‡b GKwU Zvjve× e‡· / K‡›UBbv‡ii wfZ‡i 01 eQi msi¶Y Ki‡Z n‡e|

- msiw¶Z wb‡Wj wbivc` Dcv‡q Dispose Ki‡Z n‡e Ges Disposed Gi jM †iKW©  ivL‡Z n‡e|

fv½v wb‡Wj wbqš¿b bxwZt

- fv½v wb‡Wj Gi †¶‡Î wb‡Wj cwieZ©‡bi Av‡M fv½v wb‡Wj Gi mg¯Í Ask Aek¨B Luy‡R †ei Ki‡Z n‡e|

- hw` fv½v wb‡Wj Gi †Kvb Ask Luy‡R bv cvIqv hvq ZLb ‡gwk‡bi Pvwicvk  (01 wKDweK Gwiqv),  †h evwì‡ji KvR nw”Qj †mB evwÛ‡ji mKj Mv‡g©›Um c„_K K‡i cÖ‡qvR‡b n¨vÛ wb‡Wj wW‡U±i †gwkb Øviv †PK Ki‡Z n‡e|

- GiciI hw` fv½v wb‡Wj Gi †Kvb Ask Luy‡R bv cvIqv hvq Zvn‡j mg¯Í evwÛjwU ‡gUvj wW‡UKkb †gwk‡b †PK Ki‡Z n‡e|

- wb‡W‡ji UzK‡iv cvIqvi ci wb‡W‡ji mg¯Í Ask Rgv K‡i bZzb wb‡Wj msMÖn Kiv hv‡e| wKš‘ wb‡W‡ji UzKiv cvIqv bv ‡M‡j H Mv‡g©›Um c„_Kfv‡e Quarantine e‡· ivL‡Z n‡e Ges cÖ‡mm †k‡l aŸsk K‡i †dj‡Z n‡e|

- wb‡Wj wbqš¿bKvix Aek¨B GB fv½v wb‡Wj Ges G‡¶‡Î M„nxZ mKj Kvh©cÖYvjxi wjwLZ †iKW© msi¶b Ki‡eb, AZ:ci bZzb wb‡Wj Bmy¨ Ki‡eb|

- hw` †gwkb wW‡UKkb G cÖPzi fv½v wb‡Wj cvIqv hvq Zvn‡j eyS‡Z n‡e wb‡Wj wbqš¿Y c×wZwU mwVKfv‡e KvR Ki‡Q bv| †m‡¶‡Î d¨v±ix KZ…©c¶ wb‡Wj wbqš¿Y c×wZi `¶Zv ms‡kvab I cybg~©j¨vqb Ki‡eb|

- †d¬vi n‡Z msMÖnxZ mKj fv½v wb‡Wj msiw¶Z ¯’v‡b GKwU Zvjve× e‡· / K‡›UBbv‡ii wfZ‡i 06 eQi msi¶Y Ki‡Z n‡e|

- msiw¶Z wb‡Wj wbivc` Dcv‡q Dispose Ki‡Z n‡e Ges Disposed Gi jM †iKW©  ivL‡Z n‡e|

wb‡Wj wW‡UKkb c×wZt

- Baby Ges Toddler Mv‡g©›Um Gi ‡¶‡Î 44 †_‡K 182 mvBR ch©šÍ Mv‡g©›Um¸‡jvi wb‡Wjgy³Zv wbwðZ Kivi Rb¨ me©†kl myBs Acv‡ikb Gi ci Aek¨B wb‡Wj wW‡U±i †gwkb Øviv Final Check Ki‡Z n‡e|

- wb‡Wj wW‡U±i †gwkb Acv‡iUi Metal contamination detection ‡PK Ki‡eb Ges †PwKs †iKW© ivL‡eb|

- wb‡Wj wW‡U±i †gwkb Acv‡iUi‡K Aek¨B †gwkb cwiPvjbvq cÖwkw¶Z n‡Z n‡e Ges †gwkb cwi®‹vi ivL‡Z n‡e|

- ‡Kvb Mv‡g©›Um ev Mv‡g©›Um Gi †Kvb Ask hw` ‡PK Kivi ci myBs †d¬v‡i †diZ cvVv‡bv nq Zvn‡j †mUv Aek¨B c„_K GKwU e‡K&ª (jvj) ev‡· ivL‡Z n‡e hv‡Z Unchecked Mv‡g©›Um Gi mv‡_ wg‡k bv hvq|

- wb‡W‡ji UzK‡iv cvIqvi ci GKB Mv‡g©›Um ev Mv‡g©›Um Gi †Kvb Ask cybivq wb‡Wj wW‡UKUi †gwk‡b †PK Ki‡Z n‡e Ges hw` †Kvb wb‡Wj aiv bv c‡o Zvn‡j D³ Mv‡g©›UmwU wbqwgZ Mv‡g©›Um iƒ‡c MY¨ n‡e|

- ïaygvÎ †h mKj cY¨ wbwðZfv‡e †PK Kiv n‡q‡Q †m¸‡jvB c¨vwKs Kiv n‡e|

- Rejected products Gi Rb¨ GKwU mywbw`©ó Quarantine Zone _vK‡Z n‡e|

- †hmKj cY¨ clearance cvIqvi Rb¨ A‡c¶gvb Ges †h¸‡jv clearance †c‡q‡Q (OK) Zv my¯úófv‡e Marked Up ivL‡Z n‡e|

KvwUs †eøW wbqš¿b c×wZt

- cÖ‡Z¨KwU KvwUs †gwk‡bi jM wkU (Bb Ges AvDU) iÿbv‡eÿY Ki‡Z n‡e|

- KvwUs †gwkb KvR †k‡l wbw`©ô ¯’v‡b ivL‡Z n‡e|

- KvwUs †eøW cwieZ©b Kivi cÖ‡qvRb n‡j KvwUs †eøW cwieZ©b  (Replacement) †iwR÷v‡i wjwce× Ki‡Z n‡e|

- mswkøô †÷vi wefvM cyivZb ev e¨envi A‡hvM¨ KvwUs †eøW cwieZ©b Ki‡Z c~‡ei© KvwUs †eøW Rgv wb‡e| Ges Zvnvi cwie‡Z© mswkøô KvwUs wefvM‡K KvwUs †eøW cÖ`vb Ki‡e|

- evwZj ev e¨envi A‡hvM¨ KvwUs †eøW mswkøô †÷vi wefvM jKvi e‡· Zvjve× Ae¯’vq 02 eQi msiÿY Ki‡e|

- ‡Kvb evwZj KvwUs †eøW Ab¨ †Kvb Kv‡R ev Ab¨ †Kvb Drcv`b †d¬v‡i e¨envi Kiv hv‡e bv|

wb‡Wj wW‡U±i K¨vwj‡eªkb c×wZt

- wb‡Wj wW‡U±i †gwk‡bi ÎæwU aivi ¶gZv n‡Z n‡e 0.80 mm n‡Z 1.2 mm diameter Ges Gi mv‡_ ferrous metal sphere test card mieivn _vK‡Z n‡e|

- ‡gUvj wW‡U±i †gwkb Aek¨B Kb‡fqvi †eë UvB‡ci n‡e hvi mv‡_ ¯^qswµq wifvm© Ges Gjvg© wm‡÷g _vK‡e hv †Kvb Mv‡g©›UmG †gUvj contamination _vK‡j ¯^qswµqfv‡e †e‡R DV‡e|

- wb‡Wj wW‡UKkb †gwkbwU Aek¨B †Kvb MÖnb‡hvM¨ KZ…©c¶ Øviv Calibrated n‡Z n‡e Ges Zv KvR Kivi Dc‡hvMx n‡Z n‡e|

- GQvovI cÖwZ 2 N›Uv AšÍi Kgc‡¶ 1 evi Ferrous card w`‡q wb‡Wj wW‡UKUi †gwk‡bi Calibration †PK (9 c‡q›U) Ki‡Z n‡e Ges Zvi †iKW© ivL‡Z n‡e|

- †gwkb µgvMZ 2 N›Uv e¨env‡ii ci eÜ Ki‡Z n‡e Ges wKQy¶Y ci cybivq Start w`‡Z n‡e|

- hw` wW‡UKUi ‡gwk‡bi K¨vwj‡eªkb †PK fail K‡i Zvn‡j c~‡e©i ‡PKK…Z mKj cY¨ Aek¨B cybivq †gUvj wW‡U±i †gwkb Øviv Re-check Ki‡Z n‡e|

‡gUvj K‡›Uªvj c×wZt

- ‡Kvb Baby, Toddler Ges Child AvB‡Ug Mv‡g©›Um Gi (†hgbt Baby Club, Palomino and Here & There ) fuvR A_ev cÖ`k©b Kivi Rb¨ †Kvb ai‡Yi wcb e¨envi Kiv hv‡e bv|

- ‡h †Kvb cÖWv‡±i evwÛj Kiv, U¨vwKs Kiv, c¨vwKs Kivi †¶‡Î cÖWvKkb cÖ‡mm Gi †Kvb As‡k †Kvb ai‡bi †gUvj wcb, Zvi I ÷¨vcjvi e¨envi Kiv hv‡e bv|

- ‡Kvb m¨v¤új I cÖWvKkb Gwiqv‡Z ‡gUvj wcb e¨envi Kiv hv‡e bv| weKí wn‡m‡e †Uc, Mvg/Møy w÷K,  wK¬c/K¬v¤ú e¨envi Kiv †h‡Z cv‡i| Z‡e cÖWvKkb Gwiqv e¨wZZ Ab¨Î ‡gUvj wcb e¨envi Kiv hv‡e|

- hw` ‡Kvb Mv‡g©‡›Um Gi Drcv`b cÖwµqvq †Kvb †gUvj wcb e¨envi Ki‡ZB nq (†hgbt †dweªK ‡jBs GÛ KvwUs) ‡m‡¶‡Î d¨v±ixi DwPr n‡e Gi mwVK WKz‡g›U †gBb‡UBb Kiv hv ‡KvqvwjwU ÷vd ‡PK Ki‡eb|

- nvZ w`‡q myZv KvUvi †¶‡Î †Kvb jyR (†Lvjv) KvUvi / wmRvi e¨envi Kiv hv‡e bv| wb‡Ri Ges Mv‡g©‡›Um Gi wbivcËvi Rb¨ KvUvi / wmRvi Aek¨B †eu‡a KvR Ki‡Z n‡e|

- ‡gUvj wW‡U±i †gwkbB n‡e GKgvÎ cÖ‡ek c_ hvi gva¨‡g Drcvw`Z Mv‡g©›Um ÒwewQbœ avZe gy³ GjvKviÓ wfZi w`‡q c¨vwKs Gwiqvq cÖ‡ek K‡i _v‡K|

- ‡gUvj wd« Gwiqvi‡Z †Kvb wmRvi, KvUvi, †gUvj wcb, Zvi I ÷¨vcjvi e¨envi Kiv hv‡e bv|

43 Pest Control Policy (KxU cZ½ wbqš¿b bxwZ)

fywgKv t

Drcv`b KviLvbvq Kg©iZ mKj Kg©x Ges Drcvw`Z c‡b¨i Rb¨ GKwU my›`i, ¯^v¯’¨Ki I wbivc` Kg©¯’j _vKv AZ¨vek¨K| Kg©¯’‡j wewfbœ mg‡q Drcv`b K‡g© e¨eüZ wewfbœ Dcv`v‡bi AvKl©‡Y wKsev wbivc` Avkª‡qi Avkvq KviLvbvi Pvwicv‡k wePiYKvix wewfbœ ai‡bi KxU cZ½ KviLvbvi Af¨šÍ‡i cÖ‡ek K‡i _v‡K wKsev Kivi †Póv K‡i| GmKj KxU cZ½ KviLvbvi wfZ‡i `xN© mgq Ae¯’vb Ki‡j KviLvbvi Kg©cwi‡ek A-¯^v¯’¨Ki Kivmn Drcvw`Z c‡b¨i ¸YMZ gv‡bi e¨vcK ¶wZ mvab K‡i _v‡K| ZvB mKj ai‡bi ¶wZKviK KxU cZ½ wbab I wbqš¿‡Yi Rb¨ bxwZgvjv cÖYqb K‡i‡Q|

D‡Ï¨k¨ t

KviLvbvi my¯’¨ Kg©cwi‡ek Ges Drcvw`Z c‡b¨i wbivc` msi¶Y wbwðZ Kiv|

cwiwa t

KviLvbvi mKj Drcvb GjvKv, mKj G‡·mwiR I †dweªKm ¸`vg, PzovšÍ c‡b¨i ¸`vg, mKj †MvmjLvbv I †kŠPvMvi Ges KviLvbvi Pvwicvk GB KxU cZ½ wbaY I wbqš¿Y bxwZgvjvi AvIZvf~³|

wbab I wbqš¿Y wcwiqW t

¶wZKi KxU cZ½ wbqš¿b I wbab Kvh©µg ¸‡jv wbqš¿Y/wbab cÖwµqvi Dci wfwË K‡i †i¸jvi/gvwmK/lvb¥vwmK wfwË‡Z cwiPvwjZ n‡e| wbqš¿Y/wbab cÖwµqvwU cwiPvwjZ n‡e `yB fv‡et

- KviLvbvi wbR¯^ cwiPvjb c×wZ‡Zt - hv wKQy wKQy wbqš¿Y/wbab cÖwµqv †hgb- njy` Av‡jv we”Q~ibKvix evj¦, ‡c÷ wKwjs evj¦, wWUvi‡R›U I RxevbybvkK e¨env‡ii gva¨‡g wbqwgZ wfwË‡Z Ges wKQy wKQy wbqš¿Y/wbab cÖwµqv †hgb- KxUbvkK e¨env‡ii gva¨‡g gvwmK wfwË‡Z cwiPvwjZ n‡e|

- ewn¯’ cwiPvjb c×wZt hv KviLvbvi ewni¯’ †Kvb jvB‡mÝavix †c÷ K‡›Uªvj †Kv¤úvbxi gva¨‡g cÖ‡qvRbxq mKj ai‡bi KxUbvkK e¨envi I Ab¨vb¨ wbab cÖwµqvi Øviv AšÍZ cÖwZ Qq gv‡m GKevi cwiPvwjZ n‡e| D³ †c÷ K‡›Uªvj Kvh©µg Gi mvwU©wd‡KU wb‡Z n‡e Ges D³ mvwU©wd‡KU G Aek¨B e¨eüZ †Kwg‡K¨‡ii bvg D‡jøL _vK‡Z n‡e|

‡c÷ K‡›Uªvj Kvh©cÖYvjxt

KviLvbv cwiPvjbvi †¶‡Î GB bxwZgvjv AbymiY Kiv mKj ¯Í‡i eva¨ZvgyjK| ¶wZKi e¯‘ / KxUcZ½ wbqš¿b e¨envi msµvšÍ bxwZgvjv cÖbqY Ges Dnv wbb¥iƒc fv‡e Abymib Kiv nq t-

- njy` Av‡jv we”Q~ibKvix evj¦ e¨envit KviLvbvi PZzw`©‡K cÖwZwU Rvbvjvq KxUcZ½ cÖwZ‡ivaKvix ev‡j¦i e¨e¯’v Av‡Q hv‡Z mÜ¨vi ci KviLvbvi wfZ‡i KxUcZ½ cÖ‡ek Ki‡Z bv cv‡i|

- ‡c÷ wKwjs evj¦ e¨envit KviLvbvi cÖwZwU cÖavb `iRvq KxUcZ½ cÖwZ‡ivaKvix ‡c÷ wKwjs evj¦ ¯’vcb Kiv Av‡Q hv‡Z w`‡bi †ejvq wKsev mÜ¨vi ci KviLvbvi wfZ‡i KxUcZ½ cÖ‡ek Ki‡Z bv cv‡i

- KxUbvkK e¨envit gkv gvwQi esk we¯Ívi cÖwZ‡iv‡ai Rb¨ gv‡m AšÍZ c‡¶ GKevi cwi‡ek I    gvby‡li Rb¨ mnbxq gvÎvq KxUbvkK †¯cÖ Mvb Øviv KviLvbvi Pvicv‡k Ges Af¨šÍ‡i wKQz wKQz wbw`©ó ¯’v‡b I b`©gv¸wj‡Z Qwo‡q †`Iqv nq|

- wWUvi‡R›U I RxevbybvkK e¨envit cÖwZw`b Kgc‡¶ Pvievi †MvmjLvbv I †kŠPvMvi wWUvi‡R›U I RxevbybvkK Øviv cwi¯‹vi Ki‡Yi e¨e¯’v i‡q‡Q|

cÖwZw`b Kgc‡¶ `yBevi ‡fvRb K¶ wWUvi‡R›U I RxevbybvkK Øviv cwi¯‹vi Ki‡Yi e¨e¯’v i‡q‡Q|

Dc‡iv³ bxwZgvjv me©`v ‡g‡b Pjvi Rb¨ mKj ¯Í‡ii kªwgK‡`i wbqwgZ eySv‡bv n‡q _v‡K| bxwZgvjv nvjbvMv` ivLvi Rb¨ cÖ‡qvRbxq e¨e¯’v MÖnb Kiv n‡q‡Q|

44. Riæix cwiw¯’wZ e¨e¯’vcbv (Emergency Incident & Accident Management)

KviLvbv PjvKv‡j †Kvb AvKw¯§K `~N©Ubv NU‡j Ges Mv‡g©›Um ewW b÷ n‡j wb‡b¥v³ e¨e¯’v MÖnb Ki‡Z n‡e t

Amy¯’¨Zv ev Ab¨ †Kvb Kvi‡Y KvR KivKvjxb Mv‡g©›Um ewW‡Z ewg K‡i †djv t

KviLvbv PjvKvjxb †Kvb kÖwgK-Kg©Pvix kvixwiK Amy¯’¨Zvi Kvi‡Y Mv‡g©›Um ewWi Dci ewg K‡i †dj‡j cÖ_‡gB D³ kÖwgK - Kg©Pvix‡K Kv‡Ri ¯’vb †_‡K `ªæZ mwi‡q wb‡Z n‡e Ges Zvnv‡K dv÷© GBW †`evi e¨e¯Ív Ki‡Z n‡e Ges Ae¯’v ¸iæZ¡i n‡j †gwW‡K‡j wb‡Z n‡e | Zvici, D³ Kv‡Ri ¯’vbwU‡K `ªæZ Avjv`v K‡i †dj‡Z n‡e,  †h mKj Mv‡g©÷m ewW‡Z ewg †j‡M‡Q †m Mv‡g©›Um ewW¸‡jv‡K Ab¨ Mv‡g©›Um ewW I cÖWvKkb cÖ‡mm †_‡K Avjv`v K‡i Ab¨Î mwi‡q wb‡Z n‡e|  D³ mKj ewg jvMv Mv‡g©›Um ewW wbw`©ó wbqg Ges c×wZ †gvZv‡eK wWm‡cvR Gi e¨e¯Ív Ki‡Z n‡e|

†Kvb ai‡Yi kvixwiK AvNvZ ev `yN©Ubvi d‡j Mv‡g©›Um ewW‡Z i³ jvMv t

†Kvb kÖwgK-Kg©Pvixi AvNvZ ev `yN©Ubvi d‡j i³cvZ n‡j cÖ_‡gB D³ AvnZ e¨w³‡K Kv‡Ri ¯’vb mwi‡q dv÷© GBW †`evi e¨e¯’v Ki‡Z n‡e Ges Ae¯’v ¸iæZ¡i n‡j ‡gwW‡K‡j wb‡Z n‡e| †Kvb Mv‡g©›Um ewW‡Z i³ jvM‡j  D³ ¯’vbwU‡K `ªæZ Avjv`v K‡i †dj‡Z n‡e Ges †h mKj Mv‡g©÷m ewW‡Z i³ †j‡M‡Q †m Mv‡g©›Um ewW¸‡jv‡K Ab¨ Mv‡g©›Um ewW I cÖWvKkb cÖ‡mm †_‡K Avjv`v K‡i Ab¨Î mwi‡q wb‡Z n‡e| D³ mKj i³ jvMv Mv‡g©›Um ewW wbw`©ó wbqg Ges c×wZ †gvZv‡eK wWm‡cvR Gi e¨e¯Ív Ki‡Z n‡e|

†Kvb Kgx© kvixwiK Amy¯’¨Zvi Kvi‡Y Mv‡g©›Um ewW‡Z gj-g~Î Z¨vM K‡i †djv t

†Kvb kÖwgK-Kg©Pvixi kvixwiK Amy¯’¨Zvi Kvi‡Y Mv‡g©›Um ewW‡Z gj-g~Î Z¨vM K‡i †dj‡j cÖ_‡gB D³ e¨w³‡K Kv‡Ri ¯’vb mwi‡q ‡gwW‡K‡j wb‡Z n‡e| †Kvb Mv‡g©›Um ewW‡Z gj-g~Î jvM‡j  D³ Kv‡Ri ¯’vbwU‡K `ªæZ Avjv`v K‡i †dj‡Z n‡e Ges †h mKj Mv‡g©÷m ewW‡Z gj-g~Î †j‡M‡Q †m Mv‡g©›Um ewW¸‡jv‡K Ab¨ Mv‡g©›Um ewW I cÖWvKkb cÖ‡mm †_‡K Avjv`v K‡i Ab¨Î mwi‡q wb‡Z n‡e| D³ mKj gj-g~Î jvMv Mv‡g©›Um ewW wbw`©ó wbqg Ges c×wZ †gvZv‡eK wWm‡cvR Gi e¨e¯Ív Ki‡Z n‡e|

†Kvb cÖKvi Lvivc wRwb‡mi (hvnv ¯^v‡¯’i Rb¨ ÿwZKi) ms¯ú‡k© †Kvb Mv‡g©›Um ewW Avmv‡j ÿwZKi Gi aib/gvÎv Abyhvqx e¨e¯’vcbv KZ…©c‡ÿi wm×všÍ †gvZv‡eK cÖ‡qvRbxq Kvh©Ki e¨e¯’v MÖnb Ki‡Z n‡e | †Lqvj ivL‡Z n‡e ‡hb †Kvb cÖKvi Lvivc Ges ¯^v‡¯’¨i Rb¨ ûgwK Gai‡Yi †Kvb e¯‘ ev wRwbm Mv‡g©›Um ewWi mv‡_ †j‡M bv _v‡K ev wkc‡g›U Gi mv‡_ P‡j bv hvq |

45. Product safety, legality and Quality issues

If any situation occurs in aspect of Product safety, Product legality & Quality issues factory management will discuss the specific issue to concern buyer responsible & then they will seek possible solution for the specific problem. From factory end will responsible Head of Quality, Concern Merchandiser & Head of Operation.

46. Schedule Internal Audit :

Factory management will have to conduct internal audit in respect of Product Safety/Technical (Quality) & Environment issues. Local & international Laws / Rules / Regulation / Standard, Buyre Code of conduct, Buyers specific Quality Requirement, Company Quality Policy & procedure & Best practice will have to take into consideration in the internal audit. Internal Audit should have to conduct in periodic basis. The duration of the internal audit will be once in every Three month. The internal audit will be conducted by Head of Quality & Sr. Manager (Compliance) who has sound knowledge regarding Quality policy & procedure. In this audit major social compliance issue will be taken into the report.The audit will have to conduct both by on Palnt & Process visit & document review.  Also may worker/management interview will conduct if auditor consider. The report of internal audit has to communicate with all responsible section head.

47. Management Review Meeting on Quality Management System :

Factory management will have to conduct Management review meeting on Quality Management System once in every year. Next QMS review meeting date/time will have to fix in the last review meeting.  In the review meeting all department head will seat together & have to to discuss regarding last / previous year quality performance, buyer requirement, if any gap/change in the quality manual, any complain from buyer & next year quality plan / program  etc.  In the QMS review meeting must have to present Operation Head, Head of Quality, HR & Compliance Head & Merchaindising Head. Will have to prepare & formal report regarding the meeting.

48. Risk Assessment Review Meeting :

Factory management will have to conduct risk assessment review meeting on once in every year. Next risk assessment review meeting date/time will have to fix in the last review meeting.  In the review meeting all department head will seat together & have to to discuss regarding last / previous year identified risk, last year quality performance,  next year quality plan / program  etc.  In the risk assessment review meeting must have to present All department head – Quality, Production, Finishing, Washing, Compliance, Merchaindising. Will have to prepare & formal report regarding the meeting.

49. Standard Operation Procedures

1. Cutting Department Working Procedures

- Cutting Department representatives will receive fabric from the store based on the P.O. sheet and production plan requirement.

- Cutting representative must check the fabric inspection report, shade, dye lot report, shrinkage report for that particular roll.

- Adjust pattern based on the shrinkage standard prior to making cutting plan to avoid any specific issue.

- Relax the fabric roll for 12 to 24 hours based on the fabric nature and maintain relaxed report in the bin card attach on the fabric.

- Clean up the cutting table, check sharp edge around the table and attach plain marker paper on the table by gum tape to fix it properly.

- Check the marker end-to-end width with the lay fabric end-to-end width.

- Check total layer, marker length, marker placing, ply height, ends, leaning, narrow goods tension counts, fabric faults and all others and make report in cutting table checking inspection report format.

- Check the marker very carefully with the adjusted shrinkage pattern for that particular dye lot.

- Check the cutting parts in relation to pattern and make report in the same format mentioning all findings like size, parts name, Pattern variation in inch, action taken, part number & number of defected parts found.

- Maintain serial number on the cut pcs and handover to the Cutting Quality Auditor to check & verify size, fabric fault if any and the measurement of the cut pcs comparing with the actual pattern.

- Input man will then make a 25 pcs of bundle and supply to the sewing floor maintaining in a register.

2. Cutting Department Working Notes

- Check the cable and motor carefully and clean the machine prior to start of cutting.

- Check & adjust the pressure feed.

- Check the knife sharpness.

Note: Make sure all maintenance records are maintained into the machine adjustment log form.

3. Cutting Department Safety Measure

- All cutter men must wear metal gloves while using the cutting machine.

- All related persons must wear musk for dust, rush & germ free breathing.

100% KvU †c‡bj BÝ‡cKkb c×wZ

100% Cut Panel Inspection Procedure

☞.cÖ_‡g BÝ‡cKkbK…Z Lot Gi Buyer, Style Ges PO Rvb‡Z n‡e|

☞.Lot †_‡K GKwU evwÛj wb‡q evwÛj Kv‡W©i m‡½ ewW/w÷Kvi Gi Cutting number, Size, Serial, Inseam mwVK Av‡Q wK bv †`L‡Z 	n‡e|

☞.hw` me wVK _v‡K Zvn‡j evwÛj †_‡K Left Side Ges Right Side Gi GK 	wcP Cut panel wb‡q Pair (†cqvi) wVK Av‡Q wK bv †`L‡Z n‡e|

☞.hw` Dc‡iv³ c‡q‡›Ui me mwVK _v‡K Zvn‡j Cut panel check /Inspection ïiæ Ki‡Z n‡e|

☞.Cut panel check Kivi mgq hw` †h †Kvb ai‡Yi Fabrics defects cvIqv hvq Zvn‡j Zvi †iK©W ivL‡Z n‡e Ges Panel 	check reporter Gi wbKU eywS‡q w`‡Z n‡e|

☞.Cut panel check Gi †ÿ‡Î hw` †Kvb mgm¨v cvIqv hvq Zvn‡j m‡½ m‡½ Quality Superviser Gi wbKU Rvbv‡Z n‡e|

☞.Defect Cut Panel Replacement Gi mKj Kvh©µ‡gi wi‡cvU© fvjfv‡e msiÿY Ki‡Z n‡e|

wW‡d± KvU †c‡bj cÖwZ¯’vcb c×wZ

Defect Cut Panel Replace Procedure

☞.‡Kvb Cutting Gi ewW /cvm© Replace Ki‡Z n‡j, wVK H KvwUs bv¤^vi Gi cut 	piece B 100% Replace Ki‡Z n‡e|

A_©vr hw` 1 bs Cutting Gi ewW /cvm© Replace Ki‡Z nq, Zvn‡j Aek¨B 1 bs Cutting Gi cut piece B 100% Replace Ki‡Z n‡e|

☞.‡Kvb Cutting Gi ewW /cvm© Replace Ki‡Z n‡j, wVK H KvwUs bv¤^vi Gi Lay 	chart †`‡L Replace Gi Chart/Sheet mwVKfv‡e Ki‡Z n‡e|

A_©vr hw` 1 bs Cutting Gi ewW /cvm© Replace Ki‡Z nq, Zvn‡j 1 bs Cutting Gi Lay chart †`‡LB Replace Gi Chart/Sheet mwVKfv‡e Ki‡Z n‡e|

☞.Replace Gi Chart Sheet †`‡L mwVKfv‡e Roll number wgwj‡q Replace Ki‡Z n‡e|

☞.hw` †Kvb Roll Gi Same Number G Shade bv wg‡j, Zvn‡j Aek¨B wba©vwiZ Cutting (‡hgb 1 bs Cutting) Gi ‡h †Kvb Roll †_‡K Shade wgwj‡q Replace Ki‡Z n‡e|

☞.Panel wU Replace Kivi c‡i, Panel wU evwÛj Kv‡W©i/evwÛ‡ji ‡h wmwiqv‡ji wVK H wmwiqv‡j ev mwVK RvqMvq ivLv n‡”Q wKbv †`L‡Z n‡e|

☞.Dc‡ii c‡q›U ¸‡jvi †h †KvbwU‡Z mgm¨v n‡j ev mgm¨v cvIqv †M‡j m‡½ m‡½ mswkøó Quality Supervisor ‡K Rvbv‡Z n‡e|

☞.Defect Cut Panel Replacement Gi mKj Kvh©µ‡gi wi‡cvU© fvjfv‡e 	msiÿY Ki‡Z n‡e|

‡dweª· BÝ‡cKkb c×wZ

Fabrics Inspection Procedure

(AQL-4 Point System)

☞.Fabric In-House nevi ci c¨vwKs wjó Ges AW©vi wkU (Order sheet)†÷vi g¨v‡bRvi A_ev mswkøó gv‡P©ÛvBRvi Gi KvQ †_‡K msMÖn Ki‡Z n‡e |

☞.Fabric strike/Bundle  n‡Z †dweª· †idv‡iÝ bv¤^vi (Reference number )c¨vwKs wjó Gi mv‡_ wgwj‡q †`L‡Z n‡e |

☞.mewKQz wVK _vK‡j cÖ_‡g 10% nv‡i Ges ‡dBj Ki‡j 2q evi AviI 10% nv‡i †PK Ki‡Z n‡e| Gi ciI †dBj Ki‡j cieZx©‡Z 100% nv‡i Fabrics †PK Ki‡Z n‡e |

☞.†dweª· BÝ‡cKkb Gi mgq cÖwZ 10 MR (Yeard) ci ci Running Shading †PK 	Ki‡Z n‡e |

☞.hw` Running Shading cvIqv hvq Zvn‡j  Swatch †K‡U †mKkb BbPvR© Gi wbKU eywS‡q w`‡Z n‡e |

☞.†dweª· BÝ‡cKk‡b AQL-4 Point System Gi  wbqgvejx t

- 0" (k~b¨) BwÂ †_‡K 3" (wZb) BwÂ ch©šÍ  = 1 c‡q›U |

- 3" (wZb) BwÂ †_‡K 6" (Qq) BwÂ ch©šÍ   = 2 c‡q›U |

- 6" (Qq) BwÂ †_‡K 9" (bq) BwÂ ch©šÍ    = 3 c‡q›U |

- 9" (bq) BwÂ †_‡K Dc‡i                    = 4 c‡q›U |

   hw` †Kvb Hole _v‡K Zvn‡j 4 c‡q›U G  wW‡d± KvD›U Ki‡Z n‡e |

☞.Shade Shrinkage-Gi Rb¨ wb‡¤œv³fv‡e †dweª· KuvU‡Z n‡e t

- bb Iqvk (Non-Wash) †dweª· Gi Rb¨ 10% Ges

- Iqvk ‡÷ªP (Wash & Streatch) †dweª· Gi Rb¨ 100% KuvU‡Z n‡e|

☞.Shade Shrinkage Abyhvqx †dweª·‡K Avjv`v Ki‡Z n‡e|

☞.Reject †dweª· Avjv`v K‡i wba©vwiZ ¯’v‡b †iKW© mnKv‡i ivL‡Z n‡e|

☞.†dweª· B‡bm‡cKkb Gi mKj Kvh©µ‡gi wi‡cvU© fvjfv‡e msiÿY Ki‡Z n‡e|

‡dweª· †¯úªwWs BÝ‡cKkb c×wZ

Fabrics Spreading Inspection Procedure

☞.Marker wU †Kvb Buyer Ges Style Gi Zv mwVKfv‡e Rvb‡Z n‡e|

☞.Marker wU †Uwe‡ji Dc‡i weQv‡Z n‡e, Z‡e Marker wU hw` 15/16 M‡Ri †ekx nq 	Zvn‡j Aek¨B 2’’ †ewk ai‡Z n‡e| Kvib c‡i KvwUs Kivi mgq Marker Gi †P‡q K‡g †h‡Z cv‡i|

☞.‡Uwe‡ji Dci Marker weQv‡bvi ci †Uwe‡ji Marking mwVK Av‡Q wK bv †`L‡Z n‡e|

☞.‡h Buyer/Style- Gi Marker H Buyer/Style Gi Fabrics Swatch †`‡L Fabrics wVK Av‡Q wK bv †`L‡Z n‡e Ges Shade, Percent  wjLv Av‡Q wK bv †`L‡Z n‡e|

☞.Patern Gi Mv‡q Fabrics use Gi †h Percent wU †jLv Av‡Q †mB Percent Abyhvqx GKB ai‡bi Shade Gi Fabrics Lay w`‡Z n‡e|

☞.hw` †Kvb Kvi‡b GKB ai‡bi Shade Gi Fabrics Lay ‡kl bv nq, Zvn‡j 	wWfvBWvi (Divider) w`‡q m‡e©v”P 3wU Shade Gi Lay GK‡Î ‡`Iqv hv‡e, Z‡e 	†m‡ÿ‡Î mKj Lay Gi Percent GKB n‡Z n‡e|

☞.Lay ‡`Iqv ïiæ nIqvi ci 4-5 Lay n‡q †M‡j Marker wewQ‡q †`L‡Z n‡e †Uwej  	Marking ev Lay aiv mwVK Av‡Q wKbv|

☞.Marker Gi †P‡q Fabrics Gi width Aek¨B †ewk n‡Z n‡e | (†hgb: Marker 	Gi width hw` nq 58”, Fabrics Gi width n‡Z n‡e 58.5”)|

☞.Lay  PjvKvjxb mg‡q †h †Kvb ai‡Yi mgm¨v n‡j m‡½ m‡½ Quality Superviser  Gi 	wbKU Rvb‡Z n‡e|

G‡·mwiR BÝ‡cKkb c×wZ

(Accessories Inspection Procedure)

☞.Accessories ‡PK ïiæ Kivi c~‡e© Gi evqvi (Buyer), Pvjvb (Challan) Ges ÷vBj (Style) Abyhvqx †÷vi g¨v‡bRvi A_ev mswkøó gv‡P©ÛvBRvi Gi KvQ †_‡K Approve Trim card msMÖn Ki‡Z n‡e |

☞.Approve Trim card Ges dvBj †_‡K Tech Pack Gi mv‡_ wgwj‡q ‡`‡L  mKj G‡·mwiR †PK Ki‡Z n‡e|

☞.Accessories ‡PK Kivi wbqgvejx (nvi) t

- n¨vÛ U¨vM (Hand Tag)		= 100% ‡PK Ki‡Z n‡e|

- cÖvBm U¨vM (Price Tag)		= 100% ‡PK Ki‡Z n‡e|

- SzKvi U¨vM (Jocker Tag)	 = 100% ‡PK Ki‡Z n‡e|

- Bjvw÷K (Elastic) 		= 100% ‡PK Ki‡Z n‡e|

- Ab¨vb¨ mKj AvB‡Ug 		= 20 % †PK Ki‡Z n‡e|

- 20% †PK G ‡Kvb Inspection ‡dj Ki‡j 100% †PK Ki‡Z n‡e|

☞.mKj †gUvj AvB‡Ug (Metal Item) Gi wb‡Kj Nickel †U÷ Ki‡Z n‡e|

☞.mKj  Thread,  Approve Trim card Gi mv‡_ Thread Gi Shade 	wgwj‡q †PK Ki‡Z n‡e|

☞.mKj Inspection Gi wi‡cvU© ‰Zix K‡i fv‡jvfv‡e msiÿb Ki‡Z n‡e|

GgeªqWvix/wcÖ›U BÝ‡cKkb c×wZ

Embroidery/Print Inspection Procedure

☞.Embroidery/Print Gi ‡P‡Ki Rb¨ Aek¨B Buyer Style 	 Rvb‡Z n‡e|

☞.Embroidery/Print Gi Approval – G †h wWRvBb Av‡Q H 	 wWRvBb wgwj‡q mwVK fv‡e †PK Ki‡Z n‡e|

☞.‡PK Kivi mgq Aek¨B Approval mvg‡b ivL‡Z n‡e Ges cÖwZ 	 evwÛj Ly‡j Approval Gi w`‡K ZvwK‡q wgwj‡q wb‡Z n‡e|

☞.evwÛj Kv‡W©i m‡½ w÷Kv‡ii Cutting number, Size, wmwiqvj mwVK Av‡Q wKbv †`L‡Z n‡e|

☞.Dc‡iv³ c‡q›U ¸‡jvi †h †KvbwU‡Z mgm¨v n‡j ev mgm¨v cvIqv 	 †M‡j m‡½ m‡½ mswkøó Quality Supervisor wbKU Rvb‡Z n‡e|

☞.Embroidery/Print BÝ‡cKk‡bi mKj Kvh©µ‡gi wi‡cvU©  fvjfv‡e msiÿY Ki‡Z n‡e|

wdDwRs ‡U÷ Gi c×wZ

Fusing Test Procedure

- Buyer approved Trim Card Gi mv‡_ Fusing wgwj‡q †`L‡Z n‡e|

- Fusing supplier Gi †`Iqv Test report Abyhvqx ‡gwk‡bi Temperature, Time, Pressure adjust K‡i wb‡Z n‡e|

- Thermo paper Øviv Before lunch - After lunch, Temperature check Ki‡Z n‡e, Stop watch Øviv Time mwVK Av‡Q wKbv †`L‡Z n‡e| †Kvb Style ïiæ n‡j Aek¨B †gwkb bZzb fv‡e †mU Ki‡Z n‡e|

- Bond Strength test machine Øviv Before lunch Ges After lunch, Bond check Ki‡Z n‡e|

- ‡Kvb mgm¨v cvIqv †M‡j ZvrÿwYK fv‡e Section In-charge †K Rvbv‡Z n‡e|

- Fusing Test Gi mKj Kvh©µ‡gi wi‡cvU© fvjfv‡e msiÿY Ki‡Z n‡e|

Process Back Part Garments Checking Procedure

cÖ‡mm d«›U cvU© Mv‡g©›Um Gi †PwKs c×wZ

Buyer: …………………….

Style: ……………………..

- Back Rise/Back Youk Gi gv_v  Up-Down Pj‡e bv |

- Youk  evKv Pj‡ebv Raw Edgeout Pj‡e bv |

- c‡KU †_‡K Youk  M¨vc †ev_ mvBW GK mgvb n‡Z n‡e |

- SKIP Broken, Joint STC Pj‡e bv |

- mg¯Í bv¤^vi GK n‡Z n‡e |

- 1/4 †mjvB †gvUv wPKb n‡j Pj‡ebv |

- c‡KU  Gi Dc‡i wWRvBb `yB cvk GK iKg n‡Z n‡e|

- myZv wb‡Wj  20/4 †evweb 20/2  |

- SPI n‡e (7)

Process Inside Garments Checking Procedure

cÖ‡mm BbmvBW Mv‡g©›Um Gi †PwKs c×wZ

Buyer: …………………….

Style: ……………………..

- W/B  Gi PIiov n‡e 1.¾”| W/B Gi cuvPwU gvK© Aek¨B wgj _vK‡Z n‡e ,Dc‡ii Ges bx‡Pi je GK mgvb n‡Z n‡e|

- Skip Broken  Joint    STC Pj‡ebv|

- †Kvb i‡qR †ei n‡j Pj‡ebv , W/B  Gi bv¤^vi ewWi mv‡_ wgj _vK‡Z n‡e|

- ewWi Stiker mvBR Gi mv‡_ †j‡e‡ji mvBR wgjv‡Z n‡e|

- P.O  †j‡ej wVK g‡Zv Av‡Q wKbv †`L‡Z n‡e|

- †j‡e‡ji cwRkb W/B  †_‡K 3Ó wb‡P n‡e|

- SPI n‡e (7) myZv, wb‡Wj 20/4 †evweb  20/2 |

- W/B  ‡g‡c †PK Ki‡Z n‡e|

Top Side of Complete Garments Checking Procedure

KgwcøU Mv‡g©›Um Gi Uc mvBW †PwKs c×wZ

Buyer: …………………….

Style: ……………………..

- W/B gvc‡Z n‡e | evi‡UK ¸‡jv Mockup Gi g‡Zv n‡”Q wKbv †PK Ki‡Z n‡e, Lop evKv n‡j Pj‡ebv|

- Front Gi Lop Wej †mjvB 20/4 Ges 20/2 n‡Z n‡e|

- W/B gvD_ evKv Pj‡e bv Ges Vp- Down Pj‡e bv|

- Mouth Close Gi Joint  ½  n‡Z n‡e |

- Skip, Broken, myZv, Sticker mn ewW Pj‡ebv |

- BTM ‡gvUv wPKb Pj‡ebv , cvKvwis Pj‡ebv  Joint stitch  ½  n‡Z n‡e|

- Side PKO Opning  †QvU Ÿo Pj‡ebv |

- Front Gi Lop Gi †Mv‡ìb myZv evwn‡i _vwK‡e|

- BK cvU© Gi Lop †Mv‡ìb †mjvB nv‡Zi evg mvB‡W  n‡e|

- SPI n‡e (7)

- myZv I wb‡Wj 20/4 Ges Bobin n‡e 20/2

- jyc  BTK  myZv wbwWj Ges †evweb  20/2 Cots

AvDUcyU †KvqvwjwU †PK Gi ÷¨vÛvW© Acv‡iwUs c×wZ (SOP)

Standard Operation Procedure (SOP) of End line Quality table

- ivwbs ÷vBj Gi c~Y© dvBj memgq ‡Uwe‡j ev nv‡Zi Kv‡Q ivL‡Z n‡e| (Complete style file for every running style have to be kept in table or near of hand for always time)

- ivwbs ÷vBj Gi Aby‡gvw`Z m¨v¤új, wUªg KvW©, †gRvi‡g›U mxU AvDUcyU †Uwe‡j Szjv‡bv _vK‡e| (Approve sample, Trim card, Measurement sheet of running style would be displayed)

- cÖwZwU evwÛj n‡Z ‡LqvjLywkgZ wKQy Mv‡g©›Um Gi cÖavb cÖavb cÖ‡mm gvc‡Z n‡e| (Measurement of main process has to be checked of garments randomly  in every bundle)

- cÖwZwU evwÛ‡ji mKj Mv‡g©›Um Gi ‡MUAvc 100% †PK Ki‡Z n‡e| (Getup of all garments has to checked 100% in every bundle)

- cÖwZwU evwÛ‡ji mKj Mv‡g©›Um Gi bvKvUv myZv Ges ÷xKvi 100% †PK Ki‡Z n‡e| (Uncut thread and sticker of all garments has to checked 100% in every bundle)

- cÖwZwU evwÛ‡ji mKj Mv‡g©›Um Gi ‡eªv‡Kb w÷P, ¯‹xc w÷P, jyR †Ubkb Av‡Q wKbv Zv 100% †PK Ki‡Z n‡e| (Broken stitch, Skip stitch, Lose  tension of all garments has to checked 100% in every bundle)

- mKj `iKvix ‡iKW© cÖwZ N›Uvq AvcUz‡WU ivL‡Z n‡e Ges DaŸ©Zb‡K †`Lv‡Z n‡e| (All the necessary documents have to filled in every hour and show to senior)

- Aëvi nvi †ekx n‡j Zv m‡½ m‡½ jvBb Pxd Ges GwcGg †K Rvbv‡Z n‡e| Zviv wVK bv Ki‡j QM Ges PM †K Rvbv‡Z n‡e| (If alter percent comes high have to inform to senior line chief & APM and if they don’t solve inform to QM & PM)

- ‡Kvb Mv‡g©›U‡m GKwU Aëvi n‡j m¤ú~Y© ewWwU †PK Ki‡Z n‡e Ges GKvwaK Aëvi n‡j cÖ‡Z¨K Aëv‡ii RvqMvq jvj ÷xKvi jvMv‡Z n‡e| (If any fault find has to checked full garments and if find one more fault sticker has to posted in all faults)

- Mv‡g©›Um †PK K‡i OK n‡j wb‡`©wkZ ¯’v‡b QC Pass ÷xKvi A_ev KvV †cwÝj w`‡q QC Pass bv¤^vi wjL‡Z n‡e| (For OK garments post “QC Pass” sticker or write “QC Pass” by pencil in mentioned place after the checking)

cÖ‡mm †KvqvwjwU †PK Gi ÷¨vÛvW© Acv‡iwUs c×wZ (SOP)

Standard Operation Procedure (SOP) of In line Process Quality table

- ivwbs ÷vBj Gi wbav©wiZ GK ev GKvwaK cÖ‡mm †PK K‡i H cÖ‡m‡mi Aby‡gvw`Z m¨v¤ú‡ji mv‡_ wgjv‡Z n‡e|

(After checking the selected one or one more process of 	running style it has to cross checked with approved sample of 	that process)

- m~Zv, m~Zvi Kvjvi, cvU© †kwWs, Gm wc AvB wVK Av‡Q wKbv †PK Ki‡Z n‡e| (Thread, Thread colour, Part shading, SPI have to be checked that is ok/not ok)

- cÖwZ N›Uvq ‡PwKs †iKW© AvcUz‡WU ivL‡Z n‡e Ges DaŸ©Zb‡K †`Lv‡Z n‡e| (All the checking record have to up-to-date in every hour and show to senior)

- ‡Kvb ev‡Û‡j GK ev GKvwaK Mv‡g©›Um Aëvi n‡j Zv ms‡kvab (†iw±dvB) K‡i c~Y© ev‡Ûj mvg‡bi †Uwe‡j cvm Ki‡Z n‡e| Am¤ú~Y© ev‡Ûj mvg‡bi †Uwe‡j cvm Kiv hv‡e bv|

(Rectify the find one or one more alters and complete bundle has 	to pass to the front table. Don’t pass incomplete bundle to front 	table)

50. PRODUCTION  PROCESS FLOW DIAGRAM

Fabrics & Accessories Storage

Fabrics &Accessories Inspection

Sample Approval

Conduct PP Meeting

Cutting

Numbering

Sewing

Eyelet

Wash. (if applicable)

DPGL-2

DWPL

Finishing & Ironing.

Manager X (HR & Comp)

Final QC Checking

Officer

HR

Officer

Welfare

Officer

Payroll (X)

Officer

Admin

Packing

HR Asst

Final Shipment

Manager X (HR & Comp)

Officer

HR

Officer

Welfare (X)

Officer

Payroll (X)

Officer

Admin (X)

Officer

HR

Officer

Welfare (X)

Officer

Payroll (X)

Officer

Admin (X)

Assembly Processes- Flow chart

Fabric inspection dept. reports to

Head of Quality. Before

Giving the approval to send the

Fabric to cutting

Quality manager coordinates with cutting,

Stitching & finishing dept. to inform them

About the critical operations and

Predictable problems in advance

Cutting dept. gets the shrinkage values

Of each lot checked and amends the patterns

Accordingly

Fabric is laid keeping in view that

All the garments come from a single lot

Of the fabric

Each layer is checked as all the layers

Should be identical to each other

Lengthwise & widthwise

Careful cutting block cutting

And re-cutting when required.

Each bundle is checked so that no faulty

Piece goes to Sewing

QC in the cutting dept. also checks

The particulars on the job card

& tallies the same

QA dept. helps the production dept. in

Setting up of checkpoints to monitor the quality of

Critical operations.

First 50-100 pcs. From each line for each

Style are thoroughly checked

Again pieces are checked for the operations

Which were found faulty in the earlier

Inspection and this process is repeated

Until fault free pieces are made

First few pieces are washed (if required) and

Ironed to see post sewing faults

During mass production, each operation is

Monitored by line-QC and bundles are checked

At regular intervals

Any operator found making repetitive mistakes

Is reported to production in charge

Immediately

Final checking of the stitched pcs. By the

Production checkers on the Production floor,

All quality parameters are checked

And reports are made.

Instructing iron men and washing in charge

About the possible improvements and techniques

To be followed as per need.

Finishing checkers checks each garment

For all specified quality parameters

Finishing QA's check the finished pcs daily

& report to Head of Quality as per required AQL level.

Pre-Final audit of each shipment, before it is

Offered to QA’S from buying houses at

Required AQL level.

Shipment sent.

QA’s conduct AQL inspection of each lot of the styles printed or embroidered for the desired AQL level, report to Quality manager which later is passed on to head of  production. Quality manager himself also goes to embroidery unit and coordinates with the dept head for overcoming the problems of embroidery. If the quality level is below than the required one then the goods are returned to the respective unit either for rectification or for total reprocessing of the order.